Gigabit Ethernet Switch ECS4155&ECS4655-30T/ ECS4155&ECS4655-30P

Phase 2 Firmware Version 2.1.10.262

INTRODUCTION:

The ECS4655-30T is a 24*2.5G RJ45 ports plus 6*25G SFP28 uplink ports L3 2.5GbE Switch.

The ECS4655-30P is a 24*2.5G RJ45 ports plus 6*25G SFP28 uplink ports L3 2.5GbE Switch.

The ECS4155-30T is a 24*2.5G RJ45 ports plus 6*25G SFP28 uplink ports L2+ 2.5GbE Switch.

The ECS4155-30P is a 24*2.5G RJ45 ports plus 6*25G SFP28 uplink ports L2+ 2.5GbE Switch.

Local Console Management (LCM) allows the user to monitor and configure the ECS4655 Series from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| EIT Testig Version | 1.1.0.244 | EIT Test Version Gitlab: Simba Main | 2024/11/27 |
| NTC Testig Version | 1.1.1.244 | NTC Testig Version Gitlab: Simba Main | 2024/12/3 |
| NTC Testig Version | 1.1.2.244 | NTC Testig Version Gitlab: Simba Main | 2024/12/5 |
| EIT Testig Version | 1.1.3.244 | EIT Test Version Gitlab: Simba Main | 2024/12/31 |
| NTC Testig Version | 1.1.4.251 | NTC Testig Version Gitlab: Simba Main | 2025/1/8 |
| EIT Testig Version | 1.1.5.251 | EIT Test Version Gitlab: Simba Main | 2025/2/7 |
| NTC Testig Version | 1.1.6.251 | NTC Test Version Gitlab: Simba Main | 2025/2/12 |
| EIT NTC Testig Version | 2.4.1.251 | EIT NTC Testig Version Gitlab: Simba Main | 2025/3/24 |
| EIT Testig Version | 2.4.2.252 | EIT Test Version Gitlab: Simba Main | 2025/5/27 |
| NTC Testig Version | 2.1.3.252 | NTC Test Version Gitlab: Simba Main | 2025/6/5 |
| EIT Testig Version | 2.1.4.253 | EIT Test Version Gitlab: Simba Main | 2025/7/3 |
| NTC Testig Version | 2.1.5.253 | NTC Test Version Gitlab: Simba Main | 2025/7/21 |
| NTC Testig Version | 2.1.6.253 | NTC Test Version Gitlab: Simba Main | 2025/8/6 |
| NTC Testig Version | 2.1.7.253 | NTC Test Version Gitlab: Simba Main | 2025/8/7 |
| NTC Testig Version | 2.1.8.253 | NTC Test Version Gitlab: Simba Main | 2025/8/26 |
| Patch Version | 2.1.9.262 | Patch Version Gitlab: Simba Main | 2026/4/30 |
| NTC Testig Version | 2.1.10.262 | NTC Test Version Gitlab: Simba Main | 2026/6/15 |
| NTC Testig Version | 2.1.11.263 | NTC Test Version Gitlab: Simba Main | 2026/8/17 |

LOADER:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Test Version | 1.0.0.1 | Gitlab: Simba Main Branch: Uboot2018-03 | 2024/11/27 |
| Test Version | 1.0.0.2 | Gitlab: Simba Main Branch: Uboot2018-03 | 2024/12/31 |
| Test Version | 1.0.0.3 | Gitlab: Simba Main Branch: Uboot2018-03 | 2025/1/8 |
| Test Version | 1.0.0.4 | Gitlab: Simba Main Branch: Uboot2018-03 | 2025/2/7 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Please see the ECS4655_FeatureSpec_20241202.xlsx

Installation and Configuration Notes:

In general, the ECS4655 L2+ Series Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4655 L2+ Series Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Loader V1.0.0.1

| 1 | ECS4655EC-35 | merge ECS4655 code to simba main |
| --- | --- | --- |

Runtime V1.1.0.244, Loader V1.0.0.1

| 1 | ECS4655-9 | 7ae2f151 | create new ppe for 4155 on Simba main branch |
| --- | --- | --- | --- |
| 2 | ECS4655-15 | db3d68a8 | linux and basic peripherial driver bring up |
| 3 | ECS4655EC-16 | 54aa7a10 | verify and debug SFP RX LOS status and port link |
| 4 | ECS4655EC-22 | 36358aa4 | Multiple Management IP Interface and SNMP and IP Clustering and UDLD and Multi-netting and CIDR and ARP and Static Route |
| 5 | - ECS4655EC-33 | 8d0d0c7d | remove unsupported parameter in CLI command media-type |
| 6 | ECS4655EC-35 | fe17689f | merge ECS4655 code to simba main |
| 7 | ECS4655EC-38 | acdb2aac | PHY 88E2580 does not support Auto-Media detect |
| 8 | ECS4655EC-40 | bc491a56 | change OID for ECS4655 |
| 9 | ECS4655EC-41 | 6591b4c5 | Change the search mode of the snmp public node: swModelNumber |
| 10 | ECS4655EC-42 | c717376a | Add profile license feature |
| 11 | ECS4655EC-45 | 55b80934 | rollback unicast strom-control workaround code |
| 12 | ECS4655EC-37 | d3e4f0cc | verify and debug RX LOS interrupt |
| 13 | ECS4655EC-43 | 8b7d590b | verify and debug USB |
| 14 | ECS4655EC-46 | 02a2f740 | power status verify and debug |
| 15 | ECS4655EC-51 | e0bc0392 | when port link status change CLI will hang a few seconds |
| 16 | ECS4655EC-52 | 5c4c9867 | Mirror：Failed to add ACL mirroring、MAC mirror and VLAN mirror |
| 17 | ECS4655EC-53 | b0e02d4d | verify and debug reset button |
| 18 | ECS4655EC-39 | a6c5373a | mvr receiver port can not receive igmp join packet during TG sending multicast data packet to source port. |
| 19 | ECS4655EC-54 | e10af3ee | ACL: update ACL TCAM entries. |
| 20 | ECS4655EC-34 | 37a982ac | igmpsnp mc entry spec size modified to 4k |
| 21 | ECS4655EC-55 | 36cc3f25 | update meter entries |
| 22 | ECS4655EC-13 | 0fb15891 | ECS4655 Upgrade openssl3.3.0 and openssh9.8 |

Runtime V1.1.1.244, Loader V1.0.0.1

| 1 | ECS4655EC-56 | d3b6ba97 | Modify ECS4655 dashboard front-panel port |
| --- | --- | --- | --- |
| 2 | ECS4655EC-57 | 7aa4b99e | change bootloader version |
| 3 | ECS4655EC-58 | 51716618 | change make_ecos_image to support ECS4655 |
| 4 | ECS4655EC-48 | 4695173d | MVR: The hw mc dummy entry didnot created in chip but simba cli can show the entry when first Multicast data packet trapped to cpu. |
| 5 | ECS4655EC-12 | c1a717cf | poe bring up and test |
| 6 | ECS4655EC-47 | a979e9f0 | thermal plan verify and debug |

Runtime V1.1.2.244, Loader V1.0.0.1

| 1 | ECS4655EC-59 | 3bc05e2e | MIB: Failed to compiled this EC4655 MIB. |
| --- | --- | --- | --- |
| 2 | ECS4655EC-61 | 6f2ae3bb | MIB: Failed to get switchMgt objects. |
| 3 | ECS4655EC-60 | 13aa2454 | TACACS: The DUT freezes and reboots 10 minutes after logging in via a TACACS account. |

Runtime V1.1.3.244, Loader V1.0.0.2

| 1 | ECS4655EC-63 | 1b86167c | [MAC-Thrashing-0020-0430] Cannot enable MAC Thrashing action |
| --- | --- | --- | --- |
| 2 | ECS4655EC-67 | b701e8df | [L2SW-0190, 0680] Port-channel Operation Speed-duplex only display 2500full |
| 3 | ECS4655EC-71 | f8c557bc | [SYSLOG-0210] The DUT has no "VLAN 100 link-up notification" Log after DUT reboot. |
| 4 | ECS4655EC-72 | a01aab92 | [IPv6-MLD-SNP-0110] TG2 and TG4 cannot receive multicast packets when I enable unknown-multicast mode as flood. |
| 5 | ECS4655EC-73 | 6bba12ce | [Mirror-PortBased_old-0090, 0120, 0230, 0240, 0480] I cannot configure multiple ports to mirror port. |
| 6 | ECS4655EC-75 | a01aab92 | [IGMP-Snooping-1230, 1250, 1610] Unregistered-data-flood cannot work normally. |
| 7 | ECS4655EC-77 | f8c557bc | [SNMP-0040, 0280] The DUT cannot send warmStart trap. |
| 8 | ECS4655EC-79 | 2b8d0c12 | [SNMP-0350,0360] "noauthudp-port" is wrong on SNMP Logging |
| 9 | ECS4655EC-98 | 3828c17a | [LLDP-0170, 0190] Check "PMD Auto-Negotiation Advertised Capability" was incorrect. |

Runtime V1.1.4.251, Loader V1.0.0.3

| 1 | ECS4655EC-62 | 6f2ae3bb | MIB: Failed to get/set the MIB object "dhcpRelayRestart". |
| --- | --- | --- | --- |
| 2 | ECS4655EC-64 | 6f2ae3bb | MIB: Failed to get/set the MIB object "dhcpRelayServerInetAddrEntry". |
| 3 | ECS4655EC-65 | f8c557bc | DHCP Relay: After we configure DHCP Realy; DUT return exception and system halted. |
| 4 | ECS4655EC-76 | dc5ae9c3 | MDNS: The MDNS address fails to work after waiting for four minutes. |
| 5 | ECS4655EC-78 | dc5ae9c3 | mDNS: The DUT does not respond when the mDNS query is sent from the PC. |
| 6 | ECS4655EC-80 | f8c557bc | SNMP Trap: The trap and log display coldstart when reload or web ui reset immediately |
| 7 | ECS4655EC-82 | 9d231056 | Temperature-3: Can't send rising and falling alarm to log and trap server. |
| 8 | ECS4655EC-85 | f8c557bc | SystemMgmt: DUT auto reboot after download file continuous. |
| 9 | ECS4655EC-87 | 640d339d | Exclude-vlan: The normal port cannot receive packets from uplink(Trunk port). |
| 10 | ECS4655EC-88 | f8c557bc | HTTP/HTTPs: Attempt to establish more than 5 concurrent HTTP/HTTPS sessions. The DUT will encounter an exception and reboot automatically. |
| 11 | ECS4655EC-89 | 640d339d | Exclude-vlan: DUT cannot learn any mac-address from uplink and downlink ports. |
| 12 | ECS4655EC-90 | f6895c30 | DHCPSNP: DUT doesn't reject to set circuit-id exceeds maximum 255 strings. |
| 13 | ECS4655EC-93 | f8c557bc | Syetem: The exception error occurs when executing some functions. |
| 14 | ECS4655EC-102 | cb6f5e4a | HTTP Upload: It won't pop-up error message box when load an error image. |
| 15 | ECS4655EC-104 | ff7c9c11 | DHCPSNP: The policy port setting cannot write to running setting. |
| 16 | ECS4655EC-105 | 431d6cdf | NTP: The system peer is not in peer-status. |
| 17 | ECS4655EC-108 | efd7521c | MVR6: CLI has "no mvr6 priority" command. |
| 18 | ECS4655EC-110 | 04d41032 | LLDP: some parameter setting not arrange neatly. |
| 19 | ECS4655EC-111 | 03e4cb62 | MVR6: The VLAN becomes a DVLAN in the MVR6 statistics output even if we don't configure the MVR6. |
| 20 | ECS4655EC-112 | f740423c | Copy Direction: DUT can accept non-real cfg and other file name then also can set to startup. |
| 21 | ECS4655EC-113 | ba73dfc0 | NTP: Successfully created the authentication key for index 0 from the MIB browser, but it does not appear on the CLI or web. |
| 22 | ECS4655EC-115 | 5f44b7c6 | WEB: Loss new interface-id (option 18) setting configuration page. |
| 23 | ECS4655EC-118 | 47db0261 | MVR6: The source port forwards three reports when another source port receives a report. |
| 24 | ECS4655EC-119 | dab4b0b4 | DHCPv6 Relay: DUT does not reject to set itself as destination address. |
| 25 | ECS4655EC-124 | e4610af7 | PoE: CLI cannot set the power mainpower maximum allocation. |
| 26 | ECS4655EC-125 | e4610af7 | PoE: Segmentation Fault occurs when we set PoE Maximum Available Power via WebUI. |
| 27 | ECS4655EC-126 | 26245b61 | PoE: The PoE mode button is useless. |
| 28 | ECS4655EC-129 | 3e634560 | PoE: The CLI and WebUI have different per-port power allocation settings. |
| 29 | ECS4655EC-130 |  | ERPS: After we shut down and no shutdown ring port in multi-instance; DUT still cannot forward vlan group data. |
| 30 | ECS4655EC-132 | 70453494 | PoE: The DUT is not able to supply stable power for a long time. |
| 31 | ECS4655EC-140 | bbfeed7d | L2 Switch: No command available to remove 10Half and 10Full capabilities. |
| 32 | ECS4655EC-142 | f8c557bc | MLAG: The watchdog will activate when 16K packets are injected into the MLAG environment. |
| 33 | ECS4655EC-143 | f8c557bc | MSTP: The DUTs crash when setting up the MSTP environment. |
| 34 | ECS4655EC-144 | 91635e9c | Web: The console and management port icon are overlapping |

Runtime V1.1.5.251, Loader V1.0.0.4

| 1 | ECS4655EC-176 | c861b2cf | change uboot password |
| --- | --- | --- | --- |
| 2 | ECS4655EC-66 | 0c7c1b38 | [L2SW-0160-0180, 0200, 0520-0570] Operation Speed-duplex is 100full when TG1's speed is 100half. |
| 3 | ECS4655EC-68 | 0c7c1b38 | [L2SW-0460-0530, 0560] Flow Control Type is not Back pressure when I send broadcast packets. |

Runtime V1.1.6.251, Loader V1.0.0.4

| 1 | ECS4655EC-92 | 31e9cc0a | Syetem: CLI response is very slow after rebooting the DUT. |
| --- | --- | --- | --- |
| 2 | ECS4655EC-101 | 887cc943 | FileMgmt: Perform HTTP Upload if the previous action didn't complete would cause the file system to hang. |
| 3 | ECS4655EC-105 | 0216f432 | NTP: The system peer is not in peer-status. |
| 4 | ECS4655EC-109 | 521654a3 | MVR6: The help page has a description of Forwarding Priority. |
| 5 | ECS4655EC-114 | d8cf6b97 | WEB: Loss new interface-id (option 18) setting description in Help page. |
| 6 | ECS4655EC-128 | 0d908b8 | PoE: There are some problems when the required power reaches the maximum power. |
| 7 | ECS4655EC-137 | 9b26d9e6 | LACP: The image disappears after reloading the DUT multiple times. |
| 8 | ECS4655EC-152 | 436569f6 | support dying-gasp function on craft port |
| 9 | ECS4655EC-154 | cb8d50b5 | MDNS: The DUT will no send out the update of mdns response when change the ip address from DHCP client. |
| 10 | ECS4655EC-156 | fa692aa1 | LACP: The total bandwidth value is empty when LACP is enabled on the 25G port. |
| 11 | ECS4655EC-159 | f587f7aa | PoE: The maximum power allocation on the interface is useless. |
| 12 | ECS4655EC-160 | 20837a91 | HTTP-HTTPS: Unable to display user information when accessing the web GUI. |
| 13 | ECS4655EC-164 | 8e7f05a0 | UserAuth-Local: The passwords should be compriseing at least three of key word by CLI/WEB. |
| 14 | ECS4655EC-166 | 8e7f05a0 | UserAuth-Local: The minimum length is not lower than 8 characters by CLI/WEB. |
| 15 | ECS4655EC-167 | 22d01b8f | UserAuth-Local: Customer wish to encrypt the password in the log as change guest password. |
| 16 | ECS4655EC-168 | 8d361778 | System: Segmentation Fault occurs when executing upload and download file. |
| 17 | ECS4655EC-170 | b8d91aad | MLAG:The traffic will experience blocking and flooding on other ports when reboot the DUTs. |
| 18 | ECS4655EC-171 | b8d91aad | MLAG: The traffic will experience blocking and flooding on other ports for about 60 seconds when shutdown and no shutdown the port of mlag member. |
| 19 | ECS4655EC-172 | 8d361778 | System: The LACP always is failed in system; and DUT return “Segmentation Fault”and rebooting itself. |

Runtime V2.4.1.251, Loader V1.0.0.4

| 1 | ECS4655EC-132 | 042a4a49 | PoE: The DUT is not able to supply stable power for a long time. |
| --- | --- | --- | --- |
| 2 | ECS4655EC-192 | ea29b703 | Need to update the mgmtd to the latest version V-1.8.38-17503 |
| 3 | ECS4655EC-184 | af78b626 | SelectiveQinQ: Cannot configure the double-tagged when enabled dot1q-tunnel system-tunnel-control, but reverse settings is success. |
| 4 | ECS4655EC-193 | 31eb1c9b | Dying-gasp only support management port, please remove "efm-oam" from CLI |
| 5 | ECS4655EC-150 | 09044225 | 4655 L3 function porting |
| 6 | ECS4655EC-207 | 345feabb | Traceroute: DUT cannot respond the traceroute |

Runtime V2.4.2.251, Loader V1.0.0.4

| 1 | ECS4655EC-210 | eec4f364 | Please remove "hardware profile" and "show hardware profile" command, the command is not supported for this model |
| --- | --- | --- | --- |
| 2 | ECS4655EC-219 | 2fe51cd8 | OSPFv2: DUT cannot re-learn route as remove stub network setting from all DUTs. |
| 3 | ECS4655EC-220 | f304b039 | OSPFv2: Remove vortual-link setting; DUT still learn route from virtual-link. |
| 4 | ECS4655EC-222 | 639cdada | OSPFv2: After we reload DUT and reset network type to broadcast from point-to-point. It prints error messages and ospf operation is failed. |
| 5 | ECS4655EC-224 | 4eb947b3 | OSPFv2: DUT does not redistribute connected routes on cold start-up if virtual-link is enabled. |
| 6 | ECS4655EC-227 | c1e73629 | DyingGasp: The dying-gasp information should be removed from the OAM help page. |
| 7 | ECS4655EC-228 | 2a52925c | IPv4-StaticRoute: The nullInterface cannot be added via WebUI. |
| 8 | ECS4655EC-229 | 40c5661d | SNMP: An exception occurs when snmpwalk layer3Mgt (1.3.6.1.4.1.259.10.1.64.1.18) |
| 9 | ECS4655EC-230 | 6d5f44df | FAN's RPM calculation formula changed. |
| 10 | ECS4655EC-232 | 35594ef7 | OSPFv2: Remove area 0.0.0.0 in any VLAN; DUT delete all OSPF VLAN setting. |
| 11 | ECS4655EC-234 | 40faf6b6 | PIM: DUT cannot select correct RPF neighbor sometimes. |
| 12 | ECS4655EC-235 | 32b06158 | BGP: The DUT will pop "% Unknown command:" message when disable the neighbor interface. |
| 13 | ECS4655EC-238 | c699cc02 | IGMP: Multicast data cannot be forwarded when receiving the IGMPv1 report. |
| 14 | ECS4655EC-242 | c699cc02 | IGMP: DUT sends IGMPv1 report when the interface receives IGMPv1 query. |
| 15 | ECS4655EC-244 | 0df06f5c | IGMP_MIB: The robustness can be set via MIB. |
| 16 | ECS4655EC-245 | 3d7c282d | OSPF: The maximum dynamic route entry number is incorrect. |
| 17 | ECS4655EC-249 | bc1fa586 | Config: It's failed to reset configuration when change the PVID. |
| 18 | ECS4655EC-251 | ddb0772b | IGMP_Web: The problem of setting IGMP version and robustness variable. |
| 19 | ECS4655EC-254 | ddb0772b | IGMP_Help: There is no web help page for IGMP |
| 20 | ECS4655EC-255 | 7441eb55 | IGMP: The groups can be added when adding static groups exceeds the maximum number. |
| 21 | ECS4655EC-256 | ddb0772b | WEB: DUT doesn't support this OSPF and PIM description in Help page. |
| 22 | ECS4655EC-257 | 0c823a46 | WEB: DUT cannot display “LinkStateSummary”, “LinkStateASBRSummary” and “LinkStateExternalList” information. |
| 23 | ECS4655EC-258 | 2d24111b | WEB: Failed to add redistribute route. |
| 24 | ECS4655EC-261 | 4b5e41eb | PBR: Partial PBR configuration disappeared after saving and restarting. |
| 25 | ECS4655EC-262 | 9e13d11a | PBR: CLI is not responding and SW watchdog timedout if dscp 0 is set. |
| 26 | ECS4655EC-263 |  | Static LACP not removed correctly and admin-key anomaly |
| 27 | ECS4655EC-264 | 31e9224b | VRRP: The priority of VRRP is incorrect value: 281470681743460. |
| 28 | ECS4655EC-266 | c4b3ae37 | PBR: The route-map deny action would drop incoming packets. |
| 29 | ECS4655EC-268 | ae4ef876 | PBR: Packets cannot be forwarded if the first next-hop is down but the second next-hop is up. |
| 30 | ECS4655EC-284 | aac8e4c2 | OSPF: After add area to DUT by WEB; CLI halted and rebooting itself. |
| 31 | ECS4655EC-286 | 011bfc3b | RIP: When receiving a 0.0.0.0 route with a subnet mask of 255.255.255.0, the DUT shall not update its routing table. However, the DUT currently does update the route table, which is incorrect behavior. |
| 32 | ECS4655EC-287 | 7e6e9306 | RIP: Cannot learn the maximum number of RIP routes as expected. |
| 33 | ECS4655EC-288 | f0757415 | RIP: The DUT cannot counter the RecBad Packets when send the invalid RIP version. |
| 34 | ECS4655EC-289 | 8ffcb1a1 | The DUT is unable to send metric=16 (Poisoned Reverse) update packets when using the default configuration of RIPng. |
| 35 | ECS4655EC-293 | dbb8c519 | SelectiveQinQ: The DUT will be freezing and reboot when configure the selective replace and change to port-chanel. |
| 36 | ECS4655EC-239 |  | IGMP: The maximum number of multicast groups is not 4096. |

Runtime V2.1.3.252, Loader V1.0.0.4

| 1 | ECS4655EC-315 | a31d885a | OSPFv3: It always failed to set default-information originate in global setting. |
| --- | --- | --- | --- |
| 2 | ECS4655EC-317 | 5e051af4 | [IGMP-Snooping-0050] The default igmp snooping version is not 2. |
| 3 | ECS4655EC-318 | 5e051af4 | [IGMP-Snooping-0060] IGMP snooping version cannot be modified correctly |
| 4 | ECS4655EC-319 | ac7a86bb | OSPFv2: Failed to write some viryual-link timer setting to running configuration. |
| 5 | ECS4655EC-320 | 83db4343 | Function license check for L3 functions - RIPng, VRRP, PBR and UDP helper |
| 6 | ECS4655EC-321 | 89a382fe | Function license check for L3 functions - OSPF/OSPFv3/BGP/null interface/loopack interface |
| 7 | ECS4655EC-322 | 89a382fd | Function license check for L3 functions - IGMPv2/IGMPv3/PIM SM/SSM |

Runtime V2.1.4.253, Loader V1.0.0.4

| 1 | ECS4655EC-243 | a71fdac1 | IGMP: The DUT still sends a G.S query after receiving the join report again. |
| --- | --- | --- | --- |
| 2 | ECS4655EC-324 | 8b7c2969 | OSPFv2: Failed to write redistribute with tag setting to running configuration. |
| 3 | ECS4655EC-325 | e7cd5f2e | OSPFv2: Failed to write some interface setting to running configuration. |
| 4 | ECS4655EC-327 | cd863f58 | WEB: DUT not display PIM interface setting information. |
| 5 | ECS4655EC-328 | baa82c07 (OSPF) 95bdcc82 (OSPF6) | OSPFv2/V3: The warming message is incorrect as global enable OSPF with inactived status. |
| 6 | ECS4655EC-329 | 95bdcc82 | OSPFv3: The warming message is incorrect as configure OSPF interface setting under inactived status. |
| 7 | ECS4655EC-330 | 3bdaabc0 | PIM: The warming message is incorrect as configure the PIM setting under inactived status. |
| 8 | ECS4655EC-331 | 2cadd44e | #41711 It's stuck for 5 minutes while doing show running config with specific configure |
| 9 | ECS4655EC-333 | a80b26a4 | CLI: Some unexpected messages are displayed when the DUT is reloaded. |
| 10 | ECS4655EC-334 | 97f355e7 | PBR: The DSCP 0 disappears when the route-map is bound to the interface. |
| 11 | ECS4655EC-335 | 58492028 | PBR: The DSCP value changes when incoming packets match the route-map deny type sequence. |
| 12 | ECS4655EC-337 | adf64777 | L3-Stress_IGMP: After running IGMP for a while, the CLI hangs up. |

Runtime V2.1.5.253, Loader V1.0.0.4

| 1 | ECS4655EC-121 | 31ba7090 | Cable Diagnostic: The function doesn't work (no any action) |
| --- | --- | --- | --- |
| 2 | ECS4655EC-250 | 61f5e640 | The group mode shows Invalid when the interface receives IGMPv2 report. |
| 3 | ECS4655EC-347 |  | L3 System: Get the SNMP object “ospfLsdbEntry”; System is halted and rebooting itself. |
| 4 | ECS4655EC-348 | 6e805981 | ECS4655 After http upload new runtime, system startup failed |
| 5 | ECS4655EC-354 | a25c5dca | OSPFv2: Cannot delete ospf area in VLAN configuration. |
| 6 | ECS4655EC-355 | 5ad9cba1 | [Enhancement] Craft IP support to be set in Web |
| 7 | ECS4655EC-356 | 1d61875d | OSPFv2: Cannot delete ospf area in VLAN configuration. |
| 8 | ECS4655EC-357 | 8b7c2969 | OSPFv2: DUT lose these redistribute bgp metric and metric-type command. |
| 9 | ECS4655EC-358 | 97580db7 | OSPFv3: Remove area 0.0.0.0 in VLAN; DUT delete all OSPF area setting. |

Runtime V2.1.6.253, Loader V1.0.0.4

| 1 | ECS4655EC-362 | 63dde5cb | CableDiag: The status of cable-diagnostics is shown as NC (Not Cable), but it should be NT (Not Tested). |
| --- | --- | --- | --- |
| 2 | ECS4655EC-363 | 63dde5cb | CableDiag: The cable-diagnostics status is incorrect. It should show “NS” (Not Supported) on SFP ports. |
| 3 | ECS4655EC-367 | 57883538 | CableDiag: The connection status of cable-diagnostics always shows N/A and does not reflect the actual connection state. |
| 4 | ECS4655EC-370 | ef2f8361 | OSPFv2: The warming message is incorrect as set ip ospf message-digest-key in inactive status. |
| 5 | ECS4655EC-373 | 074ca536 | after setting the ip ssh authentication-retries, the actual value will be reduced by one time |

Runtime V2.1.7.253, Loader V1.0.0.4

| 1 | ECS4655EC-371 | 08141e50 | L3 System: Get the OSPF MIB and show running on DUT; System is halted and rebooting itself. |
| --- | --- | --- | --- |
| 2 | ECS4655EC-382 | f21fd5f7 | L3-Stree: The CLI becomes unresponsive when executing the show running-config command. |

Runtime V2.1.8.253, Loader V1.0.0.4

| 1 | ECS4655EC-339 | 2b47f476 | L3-Stree: The DUT2 and DUT3 will reboot automatically when the connection link is shutdown and recovery (no shutdown). |
| --- | --- | --- | --- |

Runtime V2.1.9.262, Loader V1.0.0.4

| 1 | ECS4655EC-452 | 2413b432 | RGnets ECS4155 when port enable 802.1x and mac-authentication should bypass LLDP and STP Packet |
| --- | --- | --- | --- |

Runtime V2.1.10.262, Loader V1.0.0.4

| 1 | ECS4655EC-645 | cc06a05e | ECS4655 Phase2 Patch for Openssh upgrade |
| --- | --- | --- | --- |
| 2 | ECS4655EC-634 | a4f5ab6e | SSH continuously displays abnormal log messages |
| 3 | ECS4655EC-644 | efa4b3d7 | Pronto Memory leak issue |

Runtime V2.1.11.263, Loader V1.0.0.4

| 1 | ECS4655EC-665 | 39a07682 | ECS4155 EC - Access level 8 privileges issue |
| --- | --- | --- | --- |
| 2 | ECS4655EC-693 | 482c3682 | ECS4155 upgrade OpenSSH to 10.4p1 and fix show ssh with MobaXterm |
| 3 | ECS4655EC-715 | 4b90e498 | Fix copy scp file watchdog reboot when SCP server unreachable |

modifications:

Loader V1.0.0.1

|  | Initial Version |
| --- | --- |

Runtime V1.1.0.244

|  | Initial Version |
| --- | --- |

Work as Designed (wAD) eprs:

Runtime V2.4.2.251, Loader V1.0.0.4

| 1 | ECS4655EC-188 | MLAG: The LACP peer-link configure of MLAG will disappear when save and reboot the DUT. |
| --- | --- | --- |
| 2 | ECS4655EC-221 | OSPFv2: Remove vortual-link setting; DUT still learn external route from different area. |
| 3 | ECS4655EC-223 | OSPFv2: Change network type to broadcast from point-to-point, DUT cannot learn route from stub network. |
| 4 | ECS4655EC-240 | PIM: The downstream status is forwarding, but DUT cannot forward multicast group to outgoing interface. |
| 5 | ECS4655EC-283 | PBR: The set command is applied when incoming packets match only one of many match criteria. |

Runtime V1.1.6.251, Loader V1.0.0.4

| 1 | ECS4655EC-155 | MDNS: Pinging the MDNS address from the Craft port was unsuccessful; however, the front port is functioning properly. |
| --- | --- | --- |
| 2 | ECS4655EC-157 | Exclude-vlan: The normal port cannot receive packets from all uplink ports. |
| 3 | ECS4655EC-158 | PoE: The behavior is incorrect when changing maximum allocation power and mode. |
| 4 | ECS4655EC-174 | NetworkAccess: The network-access MAC address table cannot learn all MAC addresses. |

Runtime V1.1.4.251, Loader V1.0.0.3

| 1 | ECS4655EC-91 | DoS-Protection: Find some vulnerabilities via Nessus scan. |
| --- | --- | --- |
| 2 | ECS4655EC-97 | Subnet VLAN: The egress port receive port VLAN with incorrect priority. |
| 3 | ECS4655EC-99 | RFC2544-Latency: The 2.5G latency is not good with compare ECS4650-54P |
| 4 | ECS4655EC-117 | Partial Config: It can't cover configuration when add to running-config(default.cfg) |
| 5 | ECS4655EC-120 | Port Security: The MAC address associated with port security should not be allowed to move to other ports. |
| 6 | ECS4655EC-122 | MAC-VLAN: The "mask" setting have some problem. |
| 7 | ECS4655EC-127 | LED: The port LED is always green when it links up. |
| 8 | ECS4655EC-134 | ERPS: DUT cannot forward maximum 30 instance VLAN data. |
| 9 | ECS4655EC-139 | L2 Switch: The link status is incorrect when the link partner's speed is set to 10 Mbps Half Duplex. |

Runtime V1.1.3.244, Loader V1.0.0.2

| 1 | ECS4655EC-66 | [L2SW-0160-0180, 0200, 0520-0570] Operation Speed-duplex is 100full when TG1's speed is 100half. |
| --- | --- | --- |
| 2 | ECS4655EC-68 | [L2SW-0460-0530, 0560] Flow Control Type is not Back pressure when I send broadcast packets. |
| 3 | ECS4655EC-69 | [L2SW-0280] Flow Control Type is not None when I endable capabilities flowcontrol |
| 4 | ECS4655EC-83 | [StormCtrl-Bcast-0040] All storm limit will be modified when I modify "Broadcast Storm Limit". |
| 5 | ECS4655EC-86 | [RateLimit-0060, 0080, 0140-0160, 0230, 0250] The Egress receive packets is worng when rate-limit output is 64 kbits/second |

Known Bugs and Issues:

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

|  | Limitation Type | Description |
| --- | --- | --- |
| 1 | Chip limitation | FDB use hash algorithm to locate needed index for the pkt. It will happen collision in some case. |
| 2 | Software limitation | For verify chip can support full multicast entry. The speed of data/control packet which used for igmp snooping learning shall not in line speed. |
| 3 | Chip limitation | Broadcast/Multicast/UnknownUnicast Storm Control use the same configuration of rate limit. |
| 4 | Software limitation | Web Authentication only support IPv4 HTTP |
| 5 | Software limitation | Due to Jquery software limitation. The web browser need IE 9+, Chrome 54+, Firefox 52+ or Opera 41+. |
| 6 | Software limitation | Support muintuple vlan tag by software, but have limitation: can not support the packet need linux kernel process, like management features, arp, telnet, ping, tracerout, routing, snmp, ipv6…etc. [ES4627MB-FLF-EC-00889] |
| 7 | Chip limitation | Dynamic QoS Assignment only support configure one acl per port: Marvell support ACL resource are finite, so the upper software limit only support configure one acl per port or per function; when mac-authentication enable Dynamic Qos Assignment Base Radius, so also only support configure one acl per port，multiple acls will authentication failure. |
| 8 | SW limitation | The Double-tag function must be used in conjunction with “switchport allow vlan add VID double-tag”, and the VID is the Native vlan for configuring the port of Double-tag function, otherwise will be affected unpredictably for this vid, carry internal vid is 0 or carry three VLAN tags. |
| 9 | Chip limitation | IPv6 net route related to mask and address formats, A mask of 32 can reach 8k, while those above 64 only have 3.5k. The FAE response is implemented like this and cannot be modified, Marvell case id: 756914 |
| 11 | Chip limitation | RJ45 port cannot support half mode. On switch (AC5X) side, the USXGMII mode does not support half-duplex mode. |
| 12 | Chip limitation | RFC2544-Latency: The 2.5G latency is not good with compare 1G port. |
| 13 | Chip limitation | if the bandwidth is too slow, like 64K, and packet size is small, ratelimit will be not accurate, errorrange over 3%. (ECS4655EC-86) |
| 14 | Chip limitation | For Mac-Vlan, priority change action will work for all packet. (ECS4655EC-97) |
| 15 | Chip limitation | USXGMII mode does not support half-duplex mode. So RJ45 port cannot support half-duplex. (ECS4655EC-66 ECS4655EC-68 ECS4655EC-139) |
| 16 | Chip limitation | All storm limit will be modified when modify one type of storm control. (ECS4655EC-83) |
| 17 | Chip limitation | mac-vlan: for tagged packets when hit the entry, vid action will not work, but priority will change by the entry. (ECS4655EC-97) |
| 18 | SW limitation | MDNS not support on craft port. (ECS4655EC-155) |

Compliance support:

| Compliance Level | Compliant |
| --- | --- |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2096 | IP FORWARD(MIB-II) | -ipForwardNumber (1) - ipCidrRouteNumber (3) - ipCidrRouteNumber (4) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2021 | RMON2-MIB | probeCapabilities (probeConfig 1) probeSoftwareRev (probeConfig 2) probeHardwareRev (probeConfig 3) probeDateTime (probeConfig 4) probeResetControl (probeConfig 5) probeDownloadFile (probeConfig 6) probeDownloadTFTPServer (probeConfig 7) probeDownloadAction (probeConfig 8) probeDownloadStatus (probeConfig 9) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2674 | Q-bridge | - dot1qBase group (1-5) - dot1qTp group (1-2) - dot1qStatic group(1) - dot1qVlan (1-5) (9-10) |
| 2925 | DISMAN-PING-MIB | -pingCtlTable (2)/pingCtlEntry -pingResultsTable (3)/pingResultsEntry -pingProbeHistoryTable (4)/pingProbeHistoryEntry |
| 2925 | DISMAN-TRACEROUTE-MIB | -traceRouteCtlTable (2) - traceRouteResultsTable (3) - traceRouteProbeHistoryTable (4) - traceRouteHopsTable (5) |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |
| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |

| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| --- | --- | --- |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |
| 3584 | SNMP-COMMUNITY-MIB | - SnmpCommunityMIBObjects - (SnmpCommunityTable (1) / snmpCommunityEntry) |

ECS4655 SERIES Private Enterprise MIB Support:

| Title | Version |
| --- | --- |
| ECS4655.mib | 202307230000Z |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

Customer Support:

By Phone:

By Email:

By Web:

By Fax: