Gigabit Ethernet Switch ECS5500-12T/12P

Phase 1 Firmware Version 1.1.9.262

INTRODUCTION:

The ECS5500-12T/12P is a 8*100/1000/1G/2.5G/5G/10G Base-T ports and 2*10G SFP and 2*10G BASE-T uplink ports L2+ Switch.

Local Console Management (LCM) allows the user to monitor and configure the ECS5500 Series from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmare downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Phase1 NTC Testing Version | 1.1.9.262 | EIT and NTC Test version Gitlab: rtk_main | 2026/6/25 |
| Phase1 NTC Testing Version | 1.1.8.253 | EIT and NTC Test version Gitlab: rtk_main | 2025/8/3 |
| Phase1 EIT&NTC Testing Version | 1.1.7.253 | EIT and NTC Test version Gitlab: rtk_main | 2025/7/11 |
| Phase1 NTC Testing Version | 1.1.6.252 | NTC Test version Gitlab: rtk_main | 2025/4/2 |
| Phase1 EIT & NTC Testing Version | 1.1.5.251 | EIT and NTC Test version Gitlab: rtk_main | 2025/3/21 |
| Phase1 NTC Testing Version | 1.1.4.251 | NTC Test Version Gitlab: rtk_main | 2025/3/7 |
| Phase1 EIT Testing Version | 1.1.3.251 | EIT Test version Gitlab: rtk_main | 2025/2/24 |
| Phase1 NTC Testing Version | 1.1.2.244 | NTC Test version Gitlab: rtk_main | 2024/12/27 |
| Phase1 EIT Testing Version | 1.1.1.243 | EIT Test version Gitlab: rtk_main | 2024/9/9 |
| Phase1 EIT Testing Version | 1.1.0.243 | EIT Test version Gitlab: rtk_main | 2024/9/3 |

LOADER:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Test Version | 0.0.2.3 | Gitlab: rtk_main Branch | 2024/9/9 |
| Test Version | 0.0.2.4 | Gitlab: rtk_main Branch | 2024/12/27 |
| Test Version | 0.0.2.5 | Gitlab: rtk_main Branch | 2025/2/24 |
| Test Version | 0.0.2.6 | Gitlab: rtk_main Branch | 2025/3/7 |
| Test Version | 0.0.2.7 | Gitlab: rtk_main Branch | 2025/7/11 |

DIAG IMAGE:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Test Version |  | Diag image |  |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Installation and Configuration Notes:

In general, the ECS5500 Series Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS5500 Series Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Runtime V1.1.9.262 Loader V0.0.2.7

|  | ECS5500EC-266 | 9b3a84a | Upgrade OpenSSH from 9.8p1 to 10.3p1 |
| --- | --- | --- | --- |
|  | ECS5500EC-264 | 66eebca2 | The session number incorrectly starts at 4 instead of 1 when connecting to the DUT via SSH or Telnet.(on phase1 branch) |

Runtime V1.1.8.253 Loader V0.0.2.7

|  | ECS5500EC-239 | 68c8c10 | mgmtd upgrade to V1.9.12-17625 |
| --- | --- | --- | --- |
|  | ECS5500EC-232 | c059552 | CableDiag: The type status is incorrect when the UTP port is open (break). |
|  | ECS5500EC-233 | 7fdfb01 | CableDiag: The connection status of cable-diagnostics always shows N/A and does not reflect the actual connection state. |
|  | ECS5500EC-225 | 1915925 | ECS5500 support usb file command |

Runtime V1.1.7.253 Loader V0.0.2.7

|  | ECS5500EC-205 | 68c8c10 | Without backdoor - MD5 Checksum File Generation. |
| --- | --- | --- | --- |
|  | ECS5500EC-215 | c059552 | Create VLAN will cause the configuration to fail when trunk ID is 27, 28 |
|  | ECS5500EC-184 | 7fdfb01 | NTP: The DUT will freezing and reboot when added and delete the ntp server more times. |
|  | ECCLOUD-5468 | a0fa418 | After downgrade device cannot register to the Cloud. |
|  | ECS5500EC-221 | a9a4f64 | upgrade SDK to V4.0.4 patch578 |
|  | ECCLOUD-5799 | 6f59c07 | Add adoptd_private functionality. |
|  | ECS5500EC-222 | e2f5aae | ECS5500 add TIP PBE |
|  | ECS5500EC-228 | d337578 | Switch will print error message like “/etc/rc: /etc/rc.accton: line 5: busybox true: not found“ when startup |
|  | ECS5500EC-227 | d942da8 | nokia enhancment: record the operation from web into log |
|  | ECS5500EC-226 | d942da8 | It is need logged if the maximum number of login attempts is exceeded |
|  | ECS5500EC-140 | a9a4f64 | Cable Diagnostics: The function doesn't work compare behavior |
|  | ECS5500EC-168 | a9a4f64 | Sometimes, the some copper port is linkup to 5G after re-plug power cable and then system boot up. |

Runtime V1.1.6.252 Loader V0.0.2.6

|  | ECS5500EC-195 | c9116ec | Without backdoor - Modified version name. |
| --- | --- | --- | --- |
|  | ECS5500EC-198 | 60cc8a0 | Cppcheck find memory leak issue. |
|  | ECS5500EC-81 | 68c8c10 | ACL: CLI cannot set a MAC ACL with the IP option. |
|  | ECS5500EC-197 | b26bafe | Add mgmtd with ECS5500 series support |
|  | ECS5500EC-201 | 466260b | remove the default snmp community （public and private） |
|  | ECS5500EC-169 | 20904c2 | The port utilization in 2.5G speed always show 100% |

Runtime V1.1.5.251 Loader V0.0.2.6

| 1 | ECS5500EC-159 | 3ddcd96 | DDM SYSLOG: The alarm and warning log is incomplete(suggest add threshold is xxx dBm and current is xxx dBm display) |
| --- | --- | --- | --- |
|  | ECS5500EC-183 | 8aef2e0 | LACP: The DUT will display a debug message when LACP is enabled, and the configuration is saved or displayed using the show running-config command. |
|  | ECS5500EC-178 | b425c1e | IPv6 Source Guard: Failed to replaced this dynamic IPv6 SG entries to static binding. |
|  | ECS5500EC-158 | c435723 | The WebUI cannot display hardware counter information of IP/IPv6 ACL. |
|  | ECS5500EC-190 | 8beb118 | System Test: The image will disappear when power off/on the DUT. |
|  | ECS5500EC-191 | 8beb118 | System Test The license will disappear when power off/on. |
|  | ECS5500EC-194 | 8beb118 | ProtocolVlan: The DUT will pop worng message when add the protocol-vlan entries |

Runtime V1.1.4.251 Loader V0.0.2.6

| 1 | ECS5500EC-152 | 8b924b0 | IPv6 Source Guard: Enable IPv6 Source Guard, DHCPv6-snp and nd-snooping at same time; DUT cannot drop invalid IPV6 packets. |
| --- | --- | --- | --- |
|  | ECS5500EC-167 | 01f6d1d | UDLD: No notification for udld Link Detectio trap. |
|  | ECS5500EC-73 | c63656a | remove recovery partition from SPI NAND Flash |
|  | ECS5500EC-94 | 874d2a9 | MIB: The mib fie is miss option by "fileAutoUpgradeMgt" |
|  | ECS5500EC-145 | 172eac6 | DiffServ: The webUI has unsupported policy configurations. |
|  | ECS5500EC-134 | 910f799 | Voice VLAN: DUT not reject to change switch port mode to Access as enable auto mode on Port. |
|  | ECS5500EC-122 | ea9ab96 | [Enhancement] Web-Auto Upgrade: It should be add option by "Automatic Opcode Upgrade Reload" |
|  | ECS5500EC-176 | a49d4e0 | Configure alias name cause CLI task crash |
|  | ECS5500EC-165 | 18e602b | The official build of 248 day LLDP issue patch |
|  | ECS5500EC-112 | 18e602b | unit of measure using wrong unit. |
|  | ECS5500EC-98 | c486b51 | IPv6-AddrConfig: The value of ND reachable time would return to the default value after setting to 0 and rebooting. |
|  | ECS5500EC-156 | 22b12f7 | MVR6: Robustness behaviour is incorrect after saving and restarting. |

Runtime V1.1.3.251 Loader V0.0.2.5

| 1 | ECS5500EC-74 | 1ad51a7 | add recovery mechanism when pressing ctrl+r in uboot |
| --- | --- | --- | --- |
|  | ECS5500EC-99 | 05764e8 | ERPS: After we inject CDP frames; this data looping occurs between 2 DUT. |
|  | ECS5500EC-83 | 747438a | Static route: DUT doesn't display default route with specific sub-mask 31 |
|  | ECS5500EC-94 | 16aba97 | IPv6-AddrConfig: DUT doesn't support 1/127 prefix. |
|  | ECS5500EC-108 | a7e0f63 | MVR: Multicast data groups cannot be cleared the by clear command. |
|  | ECS5500EC-92 | 0a7214d | IPv6-MLD-SNP: Dynamic group entries cannot be deleted when add and remove static mrouter port. |
|  | ECS5500EC-93 | cd85ed4 | IPv6-MLD-SNP: DUT doesn't forward the report to mrouter port when enable querier and proxy-reporting. |
|  | ECS5500EC-109 | 5cc9527 | MVR: The uptime of unregistered groups count. |
|  | ECS5500EC-115 | 0c00de1 | MVR6 VLAN can be overlapped. |
|  | ECS5500EC-110 | 787106d | MVR: All vlans would be considered DVLAN in mvr statistics input. |
|  | ECS5500EC-86 | 3506294 | CLI: no capabilities 10half/10full is in running-config and startup-config by default |
|  | ECS5500EC-119 | c7224d5 | add updt command to update u-boot.bix in uboot |
|  | ECS5500EC-102 | 6097d64 | IPv6-AddrConfig: The prefix length prompt message is incorrect in the ipv6 nd prefix command. |
|  | ECS5500EC-101 | 0c0de98 | IPv6-AddrConfig: The RA sent by the DUT doesn't contain prefix information of the VLAN IP address. |
|  | ECS5500EC-127 | 1645931 | TWAMP: CLI shows wrong format when the address is long IPv6 addres |
|  | ECS5500EC-85 | f161648 | AMTR: After we save and rebooting this DUT; it lose this static mac-address of LACP trunk. |
|  | ECS5500EC-148 | f161648 | LACP: Create static mac-address that will disappear after save and reboot. |
|  | ECS5500EC-142 | 45e468b | MVR6: G Query cannot count on the receiver ports. |
|  | ECS5500EC-143 | d9e4cd3 | MVR6:mvr6 port type can be set to recevier port when port mode is access. |
|  | ECS5500EC-26 | b5c4046 | Enhance the command "show tech-support" that the result needs to include all what can the user show into it |
|  | ECS5500EC-104 | b5c4046 | Copy File: Failed to copy log to TFTP server. |
|  | ECS5500EC-89 | fa387f8 | PPoE-IA: The PPPoE-IA configuration will disappear when save and reboot. |
|  | ECS5500EC-132 | 19042cf | UserAuth-Local: DUT not record this password changing log as modify user password. |
|  | ECS5500EC-121 | 4f0bbb6 | SNTP Client: The SNTP can't set ipv6 address |
|  | ECS5500EC-130 | 4c25473 | Irregular Operation: DUT can reload when firmware transmission(issue of sequence) |
|  | ECS5500EC-135 | c777a8b | SFTP_FTP_TFTP: The copy sftp/ftp/tftp add-to-running-config success, but log is not recorded. |
|  | ECS5500EC-150 | 3202ee4 | MVR6: The behavior differs from ECS4655 when the normal port, which is an MVR6 VLAN member, receives the join report. |
|  | ECS5500EC-151 | 3202ee4 | MVR6: The behavior differs from ECS4655 when the inactive receiver port, which is an MVR6 VLAN member, receives the join report. |
|  | ECS5500EC-68 | 4dfa8e3 | [IPv4-RemotePing-0100, 0110] MAXSIZE_pingPacketSize is different mibx.json |
|  | ECS5500EC-69 | 4dfa8e3 | [VLAN-MAC-Based-0060] MAX MAC VLAN Entry is different mibx.json |
|  | ECS5500EC-123 | 539b873 | MIB: The mib fie is miss option by "fileAutoUpgradeMgt" |
|  | ECS5500EC-136 | adf5c7a | Copy Direction: DUT can accept non-real cfg and other file name then also can set to startup. |
|  | ECS5500EC-125 | 423b65f | Web-File: FTP download(opcode or config file) show "Data is invalid" when first to do it. |
|  | ECS5500EC-126 | 423b65f | ConfigFileMgmt: DUT cannot copy file to FTP via WebUI after copying file to FTP via CLI. |
|  | ECS5500EC-161 | d29ac4f | design ECS5500 thermal plan |
|  | ECS5500EC-147 | 488b67f | IPv6 Source Guard: After we save running setting and rebooting this DUT; it loses all static binding entries. |
|  | ECS5500EC-163 | 5a9265d | LACP: The description of Port will change when join in the port-channel or save and reboot. |
|  | ECS5500EC-153 | 1affc08 | Selective QinQ: Selective QinQ enteris will disappear when join the the member of port-channel interface. |
|  | ECS5500EC-149 | 491b2fe | VLAN-Translation: The translation setting can't join to port channel. Note: The VLAN translation configuration method has been updated to be mutually exclusive between normal ports and trunk ID-based configurations. |
|  | ECS5500EC-146 | ea31bf7 | The source IPv6 address is incorrect when the DUT receives other queries. |
|  | ECS5500EC-158 | 8ef3fdd | ACL: The WebUI cannot display hardware counter information of IP/IPv6 ACL. |
|  | ECS5500EC-100 | 8ef3fdd | SelectiveQinQ: The configure will extra zero when create selective QinQ on CLI and Mib. |
|  | ECS5500EC-139 | f81ad77 | WEB: Loopback-detection action miss the option of bcast-discard and port-disable. |
|  | ECS5500EC-138 | 650bac1 | DHCPv6 Relay: DUT not reject to configure itself as Relay server. |
|  | ECS5500EC-131 | 3c60647 | PoE: Port1 cannot supply power and the power inline status is incorrect. |
|  | ECS5500EC-116 | ae4c45d | openssl/openssh upgrade |
|  | ECS5500EC-90 | ae4c45d | HTTP-HTTPS: Unable to access the DUT via HTTPS session |

Runtime V1.1.2.243 Loader V0.0.2.24

| 1 | ECS5500EC-59 | 29e113a | [STP-0670] "Oper Link Type" should be "Point-to-point" when port link with full-duplex. |
| --- | --- | --- | --- |
|  | ECS5500EC-25 | f28343c | Enhance the extended HTTP/HTTPS/SSH timeout logout time |
|  | ECS5500EC-60 | f3973ad | [SystemBoot] Got "Reset system error" after "copy file running" is executed |
|  | ECS5500EC-70 | 756e4ff | Update profile license feature by latest specs |
|  | ECS5500EC-71 | 6f4ce8f | Add TIP compatiblity for EC |
|  | ECS5500EC-78 | 8bd35b5 | support 5500 12T/12P web ui port panel |
|  | ECS5500EC-51 | 509500f | design NAND flash layout to store runtime and config |
|  | ECS5500EC-80 | 924c721 | modify the oid of 5500 |

Runtime V1.1.1.243 Loader V0.0.2.3

| 1 | ECS5500EC-56 | fcc7d10 | ECS5500 CLI usually fails to ping |
| --- | --- | --- | --- |
|  | ECS5500EC-55 | 17a138f | set RST0 as high to reset both of PCA9543 for HW R0B |

Runtime V1.1.0.243 Loader V0.0.2.3

| 1 | ECS5500EC-53 | 24210d9 | ECS5500 phase1 poe function debug |
| --- | --- | --- | --- |
|  | ECS5500EC-52 | 1719e64 | mac-vlan:The 11 and 12 ports of the MAC VLAN cannot be Enable |
|  | ECS5500EC-50 | ff63619 | verify and debug powersave |
|  | ECS5500EC-48 | 19ff233 | Enhance the command "port-utilization" on global config. |
|  | ECS5500EC-43 |  | IPv6 srcguard enabled, when adding a static binding entry, all IPv6 packets can go through the port. |
|  | ECS5500EC-44 | 4ab464c | verify and debug LED |
|  | ECS5500EC-37 | b96fdf | ECS5500 GPIO/RX_LOS/TX_Disable verify and debug |
|  | ECS5500EC-41 | 431ba83 | the default STP path cost is different with other brands and spec. |
|  | ECS5500EC-39 | 61c9027 | The default vlan with MVR is not same spec defines. |
|  | ECS5500EC-15 | 2d9b7a2 | Enhance DHCP option 82 setting remote ID from global mode to port interface. |
|  | ECS5500EC-22 | 4bbd0d3 | ECS5500EC-22 LACP: The ports of 5G cannot form LACP. |
|  | ECS5500EC-35 | 3c7a126 | ECS5500EC-35 VLAN: Re-enable ipv6 on VLANs, CLI print error message. |
|  | ECS5500EC-34 | 41d9e59 | CableDiag verify and debug |
|  | ECS5500EC-18 | b976577 | debug ECS5500 I2C |
|  | ECS5500EC-16 | 1e3fc4a | Need to register all tasks and threads to watchdog |
|  | ECS5500EC-32 | 2d683e4 | rtk sdk cli can not query ipmcast and mcast hw entry. |
|  | ECS5500EC-14 | bc9fbf3 | debug ECS5500 port link |

modifications:

Work as Designed (wAD) eprs:

|  | ECS5500EC-165 | WAD | PoE: Only one port can supply power if we change the power mainpower maximum allocation. |
| --- | --- | --- | --- |
|  | ECS5500EC-155 | WAD | Selective QinQ: Selective QinQ enteris of LACP will disappear when save and reboot. |
|  | ECS5500EC-141 | WAD | Loopback-detection: The recover time is incorrect from IXIA send LBD packet. |
|  | ECS5500EC-133 | WAD | PoE: Only one port can supply power if we change the power mainpower maximum allocation. |
|  | ECS5500EC-103 | WAD | ERPS: DUT cannot learn maximum VLAN mac addresses. |
|  | ECS5500EC-91 | WAD | IPv6-MLD-SNP: DUT sends the report immediately if the proxy-reporting was enabled and received a general query. |
|  | ECS5500EC-88 | WAD | CLI: The behavior of no ip default-gateway is wrong when adding default gateway via ip default-gateway and ip route command. |
|  | ECS5500EC-154 | WAD | MVR6: The DUT cannot forward the query received from the source port to other source ports. |
|  | ECS5500EC-106 | WAD | Sflow: The Polling Interval is incorrect. |

Known Bugs and Issues:

|  | ECS5500EC-82 | ACL: All ports cannot forward packets when binding the IPv6 ACL to a port egress. | EIT |
| --- | --- | --- | --- |

Any other problems than those listed above should be reported to our Technical Support Staff.

NA

Known RESTRICITIONS AND Limitations:

|  | Limitation Type | Description |
| --- | --- | --- |
| 3 | Chip limitation | ECS5500 Port 1-10 does not support forced 1000m, but only forced 100m |
| 4 | Chip limitation | RTL9310 chip does not support enabling auto negotiation for fiber port 11/12 |
| 6 | Chip limitation | RTL9310 chip known issue: sFlow packet won’t be dropped until packet buffer is used out. |
| 7 | Chip limitation | CableDiag: Cable length tolerance exceed 10 meters when length less than 10M |
| 8 | Chip limitation | TRL9310 chip did not transmit udp packets with sourceport/destination port 0 |
| 9 | Chip limitation | ACL no discards source MAC with ARP request(because of system will apply an acl to trap arp request packet to cpu,and the priority is hight than the mac acl) |
| 10 | Chip limitation | 239.x.x.x 224.0.0.x 224.0.1.x these multicast management packets cannot be considered as an unknown packets，and will be flood |
| 11 | Chip limitation | Cable test should use cat6a cable,and disable the POE function when testing |

Compliance support:

| Compliance Level | Compliant |
| --- | --- |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2096 | IP FORWARD(MIB-II) | -ipForwardNumber (1) - ipCidrRouteNumber (3) - ipCidrRouteNumber (4) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2021 | RMON2-MIB | probeCapabilities (probeConfig 1) probeSoftwareRev (probeConfig 2) probeHardwareRev (probeConfig 3) probeDateTime (probeConfig 4) probeResetControl (probeConfig 5) probeDownloadFile (probeConfig 6) probeDownloadTFTPServer (probeConfig 7) probeDownloadAction (probeConfig 8) probeDownloadStatus (probeConfig 9) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2925 | DISMAN-PING-MIB | pingCtlTable (2)/pingCtlEntry pingResultsTable (3)/pingResultsEntry pingProbeHistoryTable (4)/pingProbeHistoryEntry traceRouteCtlTable |
| 2925 | DISMAN-TRACEROUTE-MIB | -traceRouteCtlTable(2) - traceRouteResultsTable(3) - traceRouteProbeHistoryTable(4) - traceRouteHopsTable(5) |
| V-Bridge MIB | V-Bridge MIB | - dot1vProtocol group |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |

| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |
| --- | --- | --- |
| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |

ECS5500 SERIES Private Enterprise MIB Support:

| Title | Version |
| --- | --- |
| ECS5500-Series.mib | 2025030520000Z |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |