Gigabit Ethernet Switch ECS4650 Series

Phase 2 Firmware Version 2.4.8.263

INTRODUCTION:

The ECS4650-54T is a 48*1G Base-T ports and 6*25G SFP uplink ports L2+ 1GbE Switch.

The ECS4650-54P is a 48*1G Base-T ports and 6*25G SFP uplink ports L2+ 1GbE PoE Switch.

Local Console Management (LCM) allows the user to monitor and configure the ECS4650 Series from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmare downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| EIT0 Testing Version | 1.1.0.1 | EIT Test Version Gitlab: ECS4130-AC5_Phase3 Branch (2023/6/19) | 2023/6/19 |
| NTC0 Testing Version | 1.1.0.2 | NTC Test Version Gitlab: ECS4130-AC5_Phase3 Branch (2023/7/14) | 2023/7/14 |
| EIT1 Testing Version | 1.1.0.3 | EIT Test Version Gitlab: ECS4130-AC5_Phase3 Branch (2023/9/1) | 2023/9/1 |
| NTC1 Testing Version | 1.1.0.4 | NTC Test Version Gitlab: ECS4130-AC5_Phase3 Branch (2023/9/14) | 2023/9/14 |
| EIT2 Testing Version | 1.1.0.5 | EIT Test Version Gitlab: ECS4130-AC5_Phase3 Branch (2023/10/24) | 2023/10/24 |
| NTC2 Testing Version | 1.1.0.6 | NTC Test Version Gitlab: Simba_Main Branch (2023/11/2) | 2023/11/2 |
| NTC3 Testing Version | 1.1.0.7 | NTC Test Version Gitlab: Simba_Main Branch (2023/11/16) | 2023/11/16 |
| NTC4 Testing Version | 1.1.0.8 | NTC Test Version Gitlab: Simba_Main Branch (2023/12/04) | 2023/12/4 |
| NTC5 Testing Version | 1.1.0.9 | NTC Test Version Gitlab: ECS4650_Phase1 Branch (2024/01/05) | 2024/01/5 |
| NTC6 Testing Version | 1.1.0.10 | NTC Test Version Gitlab: ECS4650_Phase1 Branch (2024/06/18) | 2024/06/18 |
| NTC Testing Version | 1.1.0.11 | NTC Test Version Gitlab: ECS4650_Phase1 Branch (2024/07/08) | 2024/07/08 |
| NTC Testing Version | 1.1.0.12 | NTC Test Version Gitlab: ECS4650_Phase1 Branch (2024/07/15) | 2024/07/15 |
| EIT Testing Version | 2.4.0.252 | EIT Test Version Gitlab: Simba_Main Branch (2025/04/30) | 2025/04/30 |
| NTC Testing Version | 2.4.1.252 | NTC Test Version Gitlab: Simba_Main Branch (2025/05/16) | 2025/5/16 |
| EIT Testing Version | 2.4.2.252 | EIT Test Version Gitlab: Simba_Main Branch (2025/06/20) | 2025/06/20 |
| NTC Testing Version | 2.4.3.253 | NTC Test Version Gitlab: Simba_Main Branch (2025/07/03) | 2025/07/03 |
| NTC Testing Version | 2.4.4.253 | NTC Test Version Gitlab: Simba_Main Branch (2025/07/14) | 2025/07/14 |
| NTC Testing Version | 2.4.5.253 | NTC Test Version Gitlab: Simba_Main Branch (2025/07/17) | 2025/07/17 |
| NTC Testing Version | 2.4.6.253 | NTC Test Version Gitlab: Simba_Main Branch (2025/08/06) | 2025/08/06 |
| NTC Testing Version | 2.4.7.253 | NTC Test Version Gitlab: Simba_Main Branch (2025/08/07) | 2025/08/07 |
| NTC Testing Version | 2.4.8.263 | NTC Test Version Gitlab: DEV_ECS4650_Phase2 Branch (2026/07/21) | 2026/07/21 |

LOADER:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Test Version | 0.0.0.6 | Gitlab: ECS4130-AC5_Phase3 Branch (2023/6/19) | 2023/6/19 |
| Test Version | 0.0.0.7 | Gitlab: ECS4130-AC5_Phase3 Branch (2023/7/14) | 2023/7/14 |
| Test Version | 0.0.0.9 | Gitlab: ECS4130-AC5_Phase3 Branch (2023/9/1) | 2023/9/1 |
| Test Version | 0.0.1.0 | Gitlab: ECS4130-AC5_Phase3 Branch (2023/10/24) | 2023/10/24 |
| Test Version | 0.0.1.1 | Gitlab: Simba_Main Branch (2025/06/20) | 2025/06/20 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Please see the AC5-AC5X-Aldrin2_FeatureSpec_20231103.xlsx

Installation and Configuration Notes:

In general, the ECS4650 Series Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4650 Series Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Runtime V1.1.0.1, Loader V0.0.0.6

| 1 | NA |  |
| --- | --- | --- |

Runtime V1.1.0.2, Loader V0.0.0.7

| 1 | ECS4150EC-1 | update the guidelines of making manufactory bin file |
| --- | --- | --- |
| 2 | ECS4150EC-408 | DHCP Relay: Client failed to get Reply as it send unicast Request to DUT sometimes. |
| 3 | ECS4150AT-17 | Igmpsnooping-1060: general query suppression not work well after saveconfig&reboot |
| 4 | ECS4650EC-47 | Modify the image name for sw release |
| 5 | ECS4150AT-17 | Igmpsnooping-1060: general query suppression not work well after saveconfig&reboot |
| 6 | ECS4150EC-293 | Support MVR6 |
| 7 | ECS4650EC-43 | Create PBE of eps5520 & eps5720 for muxi customization |
| 8 | ECS4650EC-7 | Support OSPF on FRR |
| 9 | ECS4650EC-7 | Support OSPF on FRR |
| 10 | ECS4150EC-408 | DHCP Relay: Client failed to get Reply as it send unicast Request to DUT sometimes. |
| 11 | ECS4650EC-7 | Support OSPF on FRR |
| 12 | ECS4650EC-9 | Standalone & Stackable: SNMP, WEB, Download/Upgrade,L2MCAST Feature Verification |
| 13 | ECS4650EC-60 | Upgrade FRR 8.5.2 version |
| 14 | ECS4150EC-416 | Add library to parse env of u-boot |
| 15 | ECS4150EC-416 | Partition suitable for simba to parse env |
| 16 | ECS4150EC-416 | Set the baud rate when switching the startup |
| 17 | ECSS4650EC-8 | Adaptation of driver & peripheral device & PoE |
| 18 | ECS4650EC-7 | Support OSPF on FRR |
| 19 | ECS4150EC-416 | Add ignore file for libubootenv and yaml |
| 20 | ECS4650EC-7 | Support OSPF on FRR |
| 21 | ECS4650EC-57 | ATC:atc trap display unit 2 port 1 info with 57, recommand to display with unit/port. |
| 22 | ECS4150EC-416 | Fix the problem of build failed for libubootenv and yaml |
| 23 | ECS4650EC-44 | Stacking: Master and Slave would swap after save reloading |
| 24 | ECS4650EC-7 | Support OSPF on FRR |
| 25 | ECS4650EC-7 | Support OSPF on FRR |
| 26 | ECS4650EC-7 | Support OSPF on FRR |
| 27 | ECS4650EC-7 | Support OSPF on FRR |
| 28 | ECS4650EC-7 | remove virtual-link commands |
| 29 | ECS4650EC-53 | MAC-Filter-0050: network-access mod mac-authentication can be enable when port enable gust vlan |
| 30 | ECS4650EC-71 | AMTR-0050:mac-address table delete after aging time with start time as first learned. |
| 31 | ECS4650EC-79 | Support RFC 2933(IGMP SNMP) |
| 32 | ECS4650EC-71 | AMTR-0050:mac-address table delete after aging time with start time as first learned. |
| 33 | ECS4650EC-77 | BGP distance is NOT effective |
| 34 | ECS4650EC-7 | Support OSPF on FRR |
| 35 | ECS4650EC-75 | BGP Resdistribution OSPF is NOT effective |
| 36 | ECS4650EC-57 | Port on slave can not genarate temerature high alarm/warning. |
| 37 | ECS4650EC-8 | Adaptation of driver & peripheral device & PoE |
| 38 | ECS4650EC-7 | Support OSPF on FRR |
| 39 | ECS4650EC-61 | the master's stacking-port option corresponding phyport on the slave is not present |
| 40 | ECS4150IT-9 | adding the filtering feature to CLI commands of show running, show log ram and show log flash |
| 41 | ECS4650EC-87 | BGP-Accesslist[stacking]：network would missing |
| 42 | ECS4650EC-86 | BGP-1610 neighbor 192.168.1.253 timers connect 0 failed |
| 43 | ECS4650EC-8 | Adaptation of driver & peripheral device & PoE |
| 44 | ECS4650EC-94 | Migration packaging scripts and image upgrade |
| 45 | ECS4650EC-8 | Light off the stack LED during the startup process. |
| 46 | ECS4650EC-82 | BGP-0720/1490: show ip bgp prefix-list is wrong |
| 47 | ECS4650EC-8 | Implement POE stacking led function.Push button function. |
| 48 | ECS4650EC-100 | The authentication key of OSPF is wrong in show running |
| 49 | ECS4650EC-98 | Failed to set the ip OSPF priority. |
| 50 | ECS4650EC-99 | DUT1 should reject this setting router-id 0.0.0.0 |
| 51 | ECS4650EC-106 | show arp error |
| 52 | ECS4650EC-7 | Support OSPF on FRR |
| 53 | ECS4150IT-9 | adding the filtering feature to CLI commands of show running, show log ram and show log flash |
| 54 | ECS4650EC-8 | Implement POE stacking function. |
| 55 | ECS4650EC-88 | sh ip bgp community-list needs all list |
| 56 | ECS4650EC-7 | Support OSPF on FRR |
| 57 | None | modify ECS4150.mib and ECS4650.mib |
| 58 | ECS4650EC-7 | Support OSPF on FRR |
| 59 | ECS4650EC-26 | Stacking: default stacking-port option set to 2. |
| 60 | ECS4650EC-8 | In stacking mode,remove stacking line to become standalone and RJ45 led keep lighting. |
| 61 | ECS4650EC-7 | Support OSPF on FRR |
| 62 | ECS4650EC-75 | BGP Resdistribution OSPF is NOT effective |

Runtime V1.1.0.3, Loader V0.0.0.9

| 1 | ECS4650EC-272 | Mask alarm led in uboot for ecs4650 & ecs4150-54t |
| --- | --- | --- |
| 2 | ECS4650EC-271 | vtysh process reclaiming |
| 3 | ECS4650EC-266 | add stacking-port option 4 for supporting port-channel stacking port |
| 4 | ECS4650EC-265 | MAC_ACL: Access-group always bind the lastest setting |
| 5 | ECS4650EC-262 | Stacking: The DUT will overwrite the image when the slave has 2 two image. |
| 6 | ECS4650EC-260 | Web: It cannot http_upload as 2nd opcode from WebUI (when stacking) |
| 7 | ECS4650EC-259 | ImageMgmt: Copy tftp file by bad image shall be rejected |
| 8 | ECS4650EC-257 | USBdisk: Maximum length is allowed to type 128 characters, but it's only allowed to set 32 characters. |
| 9 | ECS4650EC-255 | Stacking: The stacking-port option of mib and cli that is inconsistent. |
| 10 | ECS4650EC-253 | Stacking: The DUT will pop "Segmentation Fault!" message and reboot when copy running-config to startup-config. |
| 11 | ECS4650EC-251 | WEB: Failed to enter into some function help page. |
| 12 | ECS4650EC-250 | SNMP: An error occurred when snmpwalk ipMgt. |
| 13 | ECS4650EC-249 | OSPF: Please remove unnecessary message after we configure ip address. |
| 14 | ECS4650EC-247 | UDPHelper: DHCP client can't get IP address via UDPHelper function. |
| 15 | ECS4650EC-246 | Stacking: Mirror VLAN could not mirror different unit via stacking port |
| 16 | ECS4650EC-245 | IGMP_MIB: Value of igmpCacheVersion1HostTimer is wrong. |
| 17 | ECS4650EC-244 | IGMP_MIB: Value of igmpInterfaceQuerierUpTime is wrong. |
| 18 | ECS4650EC-242 | Mirror-MAC-Based: Destination cannot mirror MAC via stacking port. |
| 19 | ECS4650EC-241 | DHCP Relay: DUT relay double DHCP Request frames to Server. |
| 20 | ECS4650EC-239 | DHCP Servwer: Failed to set option setting. |
| 21 | ECS4650EC-238 | PPPoE: PPPoE IA Remote ID cannot be deleted on CLI. |
| 22 | ECS4650EC-236 | Systemboot: Sometimes, got "APP_DEMO ERROR: [line 1238]calling cpssDxChPortSerdesSignalDetectGet returned=7, portNum=0/43". |
| 23 | ECS4650EC-235 | WEB: Lose Relay option help page. |
| 24 | ECS4650EC-233 | Mirror_ACL: Tagged packet forwarding cannot be monitored ACL by stacking. |
| 25 | ECS4650EC-232 | MIB: Cannot get/set ospf setting and Show information by MIB. |
| 26 | ECS4650EC-231 | WEB: Cannot configure OSPF setting and Show information. |
| 27 | ECS4650EC-230 | Loopback-interface: The ip address of loopback that can not become the router-id of OSPF automatically. |
| 28 | ECS4650EC-228 | Loopback-interface: The ip address of loopback that can not become the router-id of BGP automatically. |
| 29 | ECS4650EC-227 | OSPF: Failed to write ospf authentication setting to running (authentication not work). |
| 30 | ECS4650EC-226 | Web[Stacking]: Web page show 404 Not Found when change Unit ID. |
| 31 | ECS4650EC-225 | OSPFv3: After we configure stub network setting; router-id is incorrect and lose stub network setting. (Stub network cannot work) |
| 32 | ECS4650EC-224 | IGMPv3: CLI doesn't remove the group when delete the group without source address. |
| 33 | ECS4650EC-223 | IGMPv3: Last reporter and v3 expure timer is wrong when add static groups with source address. |
| 34 | ECS4650EC-221 | OSPFv3: Failed to set the metric-type of redistribute static/connect/rip. |
| 35 | ECS4650EC-220 | OSPFv3[stacking]: DUT cannot change external route metric value as set local ospf metric. |
| 36 | ECS4650EC-219 | OSPF: Failed to set the metric and metric-type of redistribute static/connect/rip. |
| 37 | ECS4650EC-218 | OSPF[stacking]: DUT cannot change external route metric value as set local ospf metric. |
| 38 | ECS4650EC-217 | OSPFv3 [stacking]: DUT is not in Stub area, but it learn neighbor from Stub area. |
| 39 | ECS4650EC-216 | OSPF [stacking]: DUT cannot learn learn route from this transit Area which is between two backbones. |
| 40 | ECS4650EC-215 | IGMPv3: Value of QRV and QQIC in igmpv3 query is wrong when chang robustval and query-interval. |
| 41 | ECS4650EC-214 | IGMPv3: Multicast data can't be forwarded when use igmpv3 report to register groups. |
| 42 | ECS4650EC-213 | IGMPv3: Last Reporter can't be showed when the interface receives igmpv3 report with include mode. |
| 43 | ECS4650EC-212 | BGP: The DUT will pop error message and the DUT will freeze. |
| 44 | ECS4650EC-211 | BGP: Thje DUT will pop error message when cnfigure the match metric to 0 |
| 45 | ECS4650EC-210 | BGP: The behavior of prefix-list is incorrect. |
| 46 | ECS4650EC-209 | BGP: Can not record the bgp event on syslog. |
| 47 | ECS4650EC-208 | VRRP: The VRRP status of DUT2 is incorrect. |
| 48 | ECS4650EC-204 | DDM: Transceiver-threshold current high-alarm is missed when show running-config and show startup-config |
| 49 | ECS4650EC-203 | MVR6: MVR6 groups can't register in normal port. |
| 50 | ECS4650EC-202 | IGMP: Dynamic groups doesn't be restricted. |
| 51 | ECS4650EC-201 | MVR6: Group in show mvr6 statistics is wrong. |
| 52 | ECS4650EC-200 | MVR6: Different domain can set the same vlan. |
| 53 | ECS4650EC-199 | MVR6: MIB doesn't support MVR6. |
| 54 | ECS4650EC-197 | MVR6_Web: Priority is not in global setting. |
| 55 | ECS4650EC-196 | Web_Help: Missing some multicast functions help page. |
| 56 | ECS4650EC-195 | RIP: Can not learning the RIP routing when save and reboot the DUT. |
| 57 | ECS4650EC-193 | OSPF]v3[stacking]: DUT write to running with incorrect ospf area=id. |
| 58 | ECS4650EC-192 | RIP: Learning the 81881 entries of route that does not keep on the route entiers |
| 59 | ECS4650EC-191 | MVR6: 802.1p Forwarding Priority is useless. |
| 60 | ECS4650EC-190 | MVR6: Multicast data groups can't be cleared. |
| 61 | ECS4650EC-189 | OSPFv3[stacking]: DUT should not learn route from non-backbone. |
| 62 | ECS4650EC-188 | MVR6: Behavior is different from MVR when the receiver port receives mld report. |
| 63 | ECS4650EC-187 | MVR6: Behavior is different from MVR when the source port receives mld query/report. |
| 64 | ECS4650EC-186 | MVR6[Stacking]: Multicast data can't forward to slave DUT's receiver ports. |
| 65 | ECS4650EC-185 | CPU-Guard[Stacking]: Current Threshold becomes 1000 packets/sec when enable and disable CPU guard. |
| 66 | ECS4650EC-184 | OSPF[stacking]: DUT send out external route with incorrect metric. |
| 67 | ECS4650EC-181 | BGP: The DUT does not rejected the setting when exceed the maximum peer-group. |
| 68 | ECS4650EC-180 | OSPF[stacking]: Failed to reset OSPF/OSPFv3 redistribute metric and metric-type value. |
| 69 | ECS4650EC-179 | PIM: Please consider to add expired timer and RFP neighbor for trouble shooting. |
| 70 | ECS4650EC-177 | OSPF [stacking]: DUT send out incorrect router-id. |
| 71 | ECS4650EC-175 | PoE[Stacking]: Behavior of PoE priority is wrong. |
| 72 | ECS4650EC-174 | OSPF [stacking]: All OSPF area setting cannot work, include stub, nssa, default-metric, authentication range. |
| 73 | ECS4650EC-173 | OSPF [stacking]: After we configure routing timers, system halted and rebooting. |
| 74 | ECS4650EC-171 | PoE[Stacking]: PoE software version of slave is wrong. |
| 75 | ECS4650EC-170 | PoE_LLDP: Power value in LLDP Extended Power-via-MDI is wrong. |
| 76 | ECS4650EC-169 | BGP: Can not set the continue of route-map. |
| 77 | ECS4650EC-168 | BGP: Delete the route-map rule, show the route-map that does not empty. |
| 78 | ECS4650EC-167 | CFM [stacking]: After we interrupt loopback test by manual, DUT still send out LBM. |
| 79 | ECS4650EC-166 | PIM: This system is halted as create 1024 multicast entries and enable igmp-snp. |
| 80 | ECS4650EC-165 | PIM: This system is halted as create 512 multicast entries and restart multicast routing. |
| 81 | ECS4650EC-164 | PIM: DUT cannot forward maximum IGMP groups data. |
| 82 | ECS4650EC-163 | PIM: DUT cannot change multicast entry status to Prune as downstream status is Null. |
| 83 | ECS4650EC-162 | PIM: DUT still forward multicast data to downstream even entry status is Null. |
| 84 | ECS4650EC-161 | PIM: Failed to clear multicast enrty. |
| 85 | ECS4650EC-160 | TWAMP: Typesetting is wrong when Sender or Reflector address is long IPv6 address. |
| 86 | ECS4650EC-159 | TWAMP: Count field value in Server Greeting packet is wrong. |
| 87 | ECS4650EC-158 | BGP: no ip extcommunity-list command. |
| 88 | ECS4650EC-157 | BGP: no ip community-list command. |
| 89 | ECS4650EC-155 | MVR6: CLI has no show debug mvr6 command. |
| 90 | ECS4650EC-154 | LLDP_help: Help page has some issues. |
| 91 | ECS4650EC-153 | LLDP-MED: CLI doesn't provide command to enable or disable MED ext-poe TLV. |
| 92 | ECS4650EC-152 | BGP: The DUT will pop message whem configure the bgp dampening. |
| 93 | ECS4650EC-151 | BGP: The DUT will pop message when added the neighbor with maximum number of remote-as. |
| 94 | ECS4650EC-150 | sdk upgrade to 4.3.11 and verification |
| 95 | ECS4650EC-148 | MVR6: Dynamic mrouter port can't be created when mvr6 source port receives tagged mld query. |
| 96 | ECS4650EC-147 | WEB: Cannot configure PIM setting by WEB. |
| 97 | ECS4650EC-146 | MIB: Cannot get Standard PIM MIB objects. |
| 98 | ECS4650EC-145 | WEB: Loss Routing Protocal Help page. |
| 99 | ECS4650EC-144 | IGMP[Stacking]: Groups can't be registered when IGMP version is 3 and receive IGMPv2 reports. |
| 100 | ECS4650EC-143 | Signature verification error when u-boot loads diag |
| 101 | ECS4650EC-142 | IGMP[Stacking]: Max Resp Time in queries is wrong when change igmp max-resp-interval. |
| 102 | ECS4650EC-141 | ECMP: DUT cannot forward all ipv6 ecmp route traffic. |
| 103 | ECS4650EC-140 | ECMP: DUT cannot forward all ipv4 ecmp route traffic. |
| 104 | ECS4650EC-139 | IGMP[Stacking]: DUT sends igmpv3 query and report when igmp version is 2. |
| 105 | ECS4650EC-138 | ECMP: Only create 6 ecmp routes, but it should support maximum 8 routing entries. |
| 106 | ECS4650EC-137 | SNTP: DUT can't accepted sntp server hostname |
| 107 | ECS4650EC-135 | IGMP[Stacking]: Group expire time is wrong when change query-interval, robustness and maximum response time. |
| 108 | ECS4650EC-134 | IGMP[Stacking]: Behavior is wrong when change robustval and query-interval value. |
| 109 | ECS4650EC-133 | IGMP[Stacking]: DUT sends igmp related packets when configure robustval value even igmp is disabled. |
| 110 | ECS4650EC-132 | IGMP[Stacking]: Multicast packets are forwarded to vlan even groups are cleared. |
| 111 | ECS4650EC-131 | IGMP[Stacking]: Groups can't register until set IGMP version again. |
| 112 | ECS4650EC-130 | WEB: Loss CFM Help page. |
| 113 | ECS4650EC-129 | PIM [stacking]: The downstream router cannot receive all multicast packets. |
| 114 | ECS4650EC-128 | IGMP: CLI shows debug messages when input multicast data. |
| 115 | ECS4650EC-127 | CFM [stacking]: MIP cannot return LBR as inject LBM with MIP target address. |
| 116 | ECS4650EC-126 | CFM [stacking]: The Up-MEP cannot return LBR as it receive untagged LBM with destination is Up-MEP. |
| 117 | ECS4650EC-125 | CFM [stacking]: DUT still send out LBM as delete down-mep under loopback test. |
| 118 | ECS4650EC-124 | CFM [stacking]: The Up-MEP cannot transmit LBM as execute loopback test. |
| 119 | ECS4650EC-123 | CFM [stacking]: The Up-MEP cannot return LTR as it receive untagged LTM with destination is Up-MEP. |
| 120 | ECS4650EC-122 | CFM [stacking]: MIP cannot return LTR as inject LTM with MIP target address. |
| 121 | ECS4650EC-121 | CFM [stacking]: After we change maximum mep_id, it cannot send out all mep CCMs. |
| 122 | ECS4650EC-119 | CFM [stacking]: Failed to return LTR from Up-MEP. |
| 123 | ECS4650EC-118 | CFM [stacking]: Failed to return LBR from normal Port. |
| 124 | ECS4650EC-116 | CLI: CLI shows debug messages when click Add/Show button of UDP Helper Address via WebUI. |
| 125 | ECS4650EC-114 | CFM [stacking]: Failed to send CCM from Up-MEP. |
| 126 | ECS4650EC-113 | UDPHelper: DUT cannot forward default udp port packets when only enable ip helper and set helper address. |
| 127 | ECS4650EC-110 | Loopback-interface: The IP address can a subnet mask to 255.255.255.254. |
| 128 | ECS4650EC-109 | IPv6-AddrConfig: Autoconfig is useless. |
| 129 | ECS4650EC-108 | System: All fan status in show system displays "Fail". |
| 130 | ECS4650EC-107 | Support PIM SNMP/WEB |
| 131 | ECS4650EC-95 | When insert SFP and fiber, it will print "ISC_PacketIncoming: receive invalid ISC packet" |
| 132 | ECS4650EC-67 | Diffserve-1140-1190: Policy Map PHB Input not work well |
| 133 | ECS4650EC-61 | the master's stacking-port option corresponding phyport on the slave is not present |
| 134 | ECS4650EC-50 | RMON:show rmon statistics only displays unit 1 intrefaces,unit 2 interfaces didnot be displayed |

Runtime V1.1.0.4, Loader V0.0.0.9

| 1 | ECS4650EC-295 | [OSPF-0330]: Special charactor can not be show in running-config |
| --- | --- | --- |
| 2 | ECS4650EC-285 | 802.1X-0330[Stacking] : MAC addresses are NOT removed after configure 802.1x port-control to auto |
| 3 | ECS4650EC-280 | VoiceVlan:port rejoin voice vlan after down/up |
| 4 | ECS4650EC-277 | When a port is power on. Change the maximum allocation to make overload. Segmentation fault will happen |
| 5 | ECS4650EC-269 | PoE: Port1-Port40 can set maximum allocation to class5-class8. |
| 6 | ECS4650EC-261 | CLI: Got message "ISC_PacketIncoming: receive invalid ISC packet!" when loopback |
| 7 | ECS4650EC-243 | MLAG: The DUT will pop error message and reboot. |
| 8 | ECS4650EC-218 | OSPF[stacking]: DUT cannot change external route metric value as set local ospf metric. |
| 9 | ECS4650EC-176 | BGP: The traffic will packet loss and pop message "ISC_PacketIncoming: receive invalid ISC packet!". |
| 10 | ECS4650EC-175 | PoE[Stacking]: Behavior of PoE priority is wrong. |
| 11 | ECS4650EC-171 | PoE[Stacking]: PoE software version of slave is wrong. |
| 12 | ECS4650EC-170 | PoE_LLDP: Power value in LLDP Extended Power-via-MDI is wrong. |
| 13 | ECS4650EC-146 | MIB: Cannot get Standard PIM MIB objects. |
| 14 | ECS4650EC-108 | System: All fan status in show system displays "Fail". |
| 15 | ECS4650EC-95 | When insert SFP and fiber, it will print "ISC_PacketIncoming: receive invalid ISC packet" |
| 16 | ECS4650EC-24 | Support IEEE 802.1ag (CFM) & Y.1731 (Ethernet OAM) |

Runtime V1.1.0.5, Loader V0.0.1.0

| 1 | ECS4650EC-345 | CFM: DUT cannot clear all defect condition even it learn maximum 32 MEPs with ccm interval=1. |
| --- | --- | --- |
| 2 | ECS4650EC-332 | Comb_MLAG: The traffic will block a few seconds when reboot the DUT. |
| 3 | ECS4650EC-331 | PIM: DUT cannot forward all multicast groups to different VLAN interface. |
| 4 | ECS4650EC-329 | IGMP: When set one static group with one source, groups source list has two entries. |
| 5 | ECS4650EC-328 | PIM: DUT still forward multicast group even no existing downstream member. |
| 6 | ECS4650EC-327 | PIM: DUT learn incorrect RPF neighbor. |
| 7 | ECS4650EC-325 | BGP: The traffic will packet block and pop message "TCN From unit ". |
| 8 | ECS4650EC-324 | MIB: Cannot get ipMRouteTable, ipMrouteInterfaceTable and ipMrouteNextHopTable objetcs. |
| 9 | ECS4650EC-321 | PIM [stacking]: All vlan interface is Prune at next time, but downstream router has igmp group member. |
| 10 | ECS4650EC-318 | OSPF: DUT return debug message as get ospf information by WEB. |
| 11 | ECS4650EC-317 | Web: Opcode upload shall be rejected directly on slave unit |
| 12 | ECS4650EC-316 | Web: Opcode is disappeared on unit 2 when visit "System > File > Show" |
| 13 | ECS4650EC-313 | OSPF (Stacking): Join DUT to NSSA network, it still learn route from neignbor router. |
| 14 | ECS4650EC-310 | IGMP: DUT sends igmp related packets when set igmp query interval and igmp is disabled. |
| 15 | ECS4650EC-308 | OSPF (Stacking): Failed to unset the OSPF authentication type. |
| 16 | ECS4650EC-305 | OSPF (Stacking): Failed to learn ospf route from stub network. |
| 17 | ECS4650EC-302 | OSPF: Failed to reset metric type of redistribute routes. |
| 18 | ECS4650EC-299 | [OSPF-0200]: the vale of Metric in ospf packet is incorrect |
| 19 | ECS4650EC-298 | [OSPF-0030]: 修改OSPF area时，cli会报错 |
| 20 | ECS4650EC-297 | [ospf-0500]: ospf 学到的路由信息不正确 |
| 21 | ECS4650EC-294 | [OSPF-0320]: The value of in NSSA area 0.0.0.1 can not be show in running-config |
| 22 | ECS4650EC-290 | [bgp-0360]： Minor error code of BGP Notification message is incorrect |
| 23 | ECS4650EC-268 | DUT can't sent OAM dying-gasp in stacking mode |
| 24 | ECS4650EC-244 | IGMP_MIB: Value of igmpInterfaceQuerierUpTime is wrong. |
| 25 | ECS4650EC-217 | OSPFv3 [stacking]: DUT is not in Stub area, but it learn neighbor from Stub area. |
| 26 | ECS4650EC-205 | DDM: Warning/alarm is only generated at 1st time |
| 27 | ECS4650EC-186 | MVR6[Stacking]: Multicast data can't forward to slave DUT's receiver ports. |
| 28 | ECS4650EC-180 | OSPF[stacking]: Failed to reset OSPF/OSPFv3 redistribute metric and metric-type value. |
| 29 | ECS4650EC-176 | BGP: The traffic will packet loss and pop message "ISC_PacketIncoming: receive invalid ISC packet!". |
| 30 | ECS4650EC-164 | PIM: DUT cannot forward maximum IGMP groups data. |
| 31 | ECS4650EC-129 | PIM [stacking]: The downstream router cannot receive all multicast packets. |
| 32 | ECS4650EC-100 | OSPF-0330[stacking]: The authentication key of OSPF is wrong in show running |

Runtime V1.1.0.6, Loader V0.0.1.0

| 1 | ECS4650EC-360 | 修复 telnet/ssh 下面的 show ip ospf 显示问题 |
| --- | --- | --- |
| 2 | ECS4650EC-359 | Fan fail on ECS4650-54T |
| 3 | ECS4650EC-356 | [OSPF-0320]: there is no configuration of translator-role of NSSA in running config |
| 4 | ECS4650EC-349 | Stacking: The file name of image will show undefined on slave unit. |
| 5 | ECS4650EC-347 | Comb_Trunk: The DUT will pop message "TCN:Master unit 1, Stacking Port 52 link-up, topology: Line." when shutdown the member of port-channel. |
| 6 | ECS4650EC-343 | Comb_MSTP: The DUT will pop message "TCN:Slave unit 2, Stacking Port 51 link-down, topology: Line." when change the priority of mstp. |
| 7 | ECS4650EC-337 | PoE[Stacking]: The slave device cannot provide power after reboot. |
| 8 | ECS4650EC-335 | System: After we enable DHCP-SNP/IPSG and inject DHCP Request; this stacking operation is failed, data looping and DUT reload itself. |
| 9 | ECS4650EC-333 | System: After we inject ARP Request and multicast streams; this stacking operation is failed, data looping and DUT reload itself. |
| 10 | ECS4650EC-326 | RIP: The traffic will packet block and pop message "TCN From unit ". |
| 11 | ECS4650EC-320 | MIB: Cannot get pimIpMRouteTable MIB objects. |
| 12 | ECS4650EC-314 | OSPF: Failed to set ospf network-type. |
| 13 | ECS4650EC-304 | PIM[Stacking]: CLI shows error messages and multicast routing table becomes empty when input multicast data. |
| 14 | ECS4650EC-303 | OSPFv3: Failed to write correct write area-id to running. |
| 15 | ECS4650EC-267 | PIM: This system is halted as disable multicast routing. |
| 16 | ECS4650EC-234 | PPPoE: The DUT will cause the TCN of stacking when inject the pppoe(PADI/PADO) packets continue. |
| 17 | ECS4650EC-171 | PoE[Stacking]: PoE software version of slave is wrong. |

Runtime V1.1.0.7, Loader V0.0.1.0

| 1 | ECS4650EC-390 | [OSPF-0500][muxi]: configuration of "area 0.0.0.1" can not be set |
| --- | --- | --- |
| 2 | ECS4650EC-389 | [OSPF-0190][muxi]: the command of "show ip ospf database external" is failed |
| 3 | ECS4650EC-387 | [BGP-0790][muxi]: The route entry in the neighbor DUT can not be learned when ospf redistributer is on |
| 4 | ECS4650EC-386 | MVR/MVR6[Stacking]: Display of port type in master device is wrong after disconnect stacking table. |
| 5 | ECS4650EC-384 | Comb_LACP: The DUT will pop message "[TCN] STKTPLG_ENGINE_Check_Stacking_Link_Down: Line" when shutdown the member of LACP. |
| 6 | ECS4650EC-383 | Comb_LACP: The DUT will pop message "[TCN] STKTPLG_ENGINE_Check_Stacking_Link_Down: Line" when shutdown the member of LACP. |
| 7 | ECS4650EC-382 | CFM: DUT cannot re-learn remote mep(cc-interval=1) as global disable and enable CFM. |
| 8 | ECS4650EC-379 | ARP: The DUT will cause the TCN of stacking when inject the ARP packets continue. |
| 9 | ECS4650EC-377 | PPPoE: The DUT will cause the TCN of stacking when inject the pppoe(PADI/PADO) packets continue. |
| 10 | ECS4650EC-376 | OSPFv3: Not write advertise summary route range to running setting. |
| 11 | ECS4650EC-375 | [IPv6-StaticRoute-0150][MUXI]: The SA is incorrect when stacking DUT forward UDP packets |
| 12 | ECS4650EC-369 | Loopback-Interface: The configure of loopback-interface that cannot keep after reboot. |
| 13 | ECS4650EC-368 | [IPv4-StaticRoute-0340][muxi]: the SA in ipv4 receved packets is incorrect |
| 14 | ECS4650EC-367 | OSPFv3 (Stacking): Failed to learn ospf route from stub network. |
| 15 | ECS4650EC-366 | [IGMP-Snooping-0950][muxi]: igmp group will be deleted when some member leaved from this group |
| 16 | ECS4650EC-365 | OSPF中相同的AREA ID无法同时在不同端口下被配置 |
| 17 | ECS4650EC-364 | OSPF[stacking]: After we reset external route metric, DUT send incorrect route metric. |
| 18 | ECS4650EC-335 | System: After we enable DHCP-SNP/IPSG and inject DHCP Request; this stacking operation is failed, data looping and DUT reload itself. |
| 19 | ECS4650EC-333 | System: After we inject ARP Request and multicast streams; this stacking operation is failed, data looping and DUT reload itself. |
| 20 | ECS4650EC-313 | OSPF (Stacking): Join DUT to NSSA network, it still learn route from neignbor router. |
| 21 | ECS4650EC-308 | OSPF (Stacking): Failed to unset the OSPF authentication type. |
| 22 | ECS4650EC-385 | [EPS5720-48T6Z] PortTrunk-0070 [stacking]: Broadcast traffic is wrong |
| 23 | ECS4650EC-371 | DDM: "!" is still existed even transceiver-monitor is disabled and clear" |
| 24 | ECS4650EC-395 | PPPoE: User cannot access the DUT from web when inject the PADI/PADO packets. |

Runtime V1.1.0.8, Loader V0.0.1.0

| 1 | ECS4650EC-418 | Stacking: The DUT will dump exception message and reboot contiune. |
| --- | --- | --- |
| 2 | ECS4650EC-413 | Some unexpected information in "show excluded-vlan" |
| 3 | ECS4650EC-411 | MVR6[stacking]: DUT forwards double mld query packets to other source and receiver ports when a source port receive one query. |
| 4 | ECS4650EC-410 | ARP: The DUT will pop interface:VLAN281470681743361 when show tech-support. |
| 5 | ECS4650EC-409 | CFM: After we re-enable CFM on DUT repeatdely, it still not learn remote meps. |
| 6 | ECS4650EC-407 | IGMP: Dynamic source address would be deleted when add one static group with source address. |
| 7 | ECS4650EC-405 | IGMP: Group mode is wrong when vlan interface receives a report with exclude mode. |
| 8 | ECS4650EC-403 | SNMP: The DUT will pop debug message "ospfd is not running" when walk the rip and ospf oid from mib. |
| 9 | ECS4650EC-402 | BGP: Cannot enable the bgp. |
| 10 | ECS4650EC-401 | OSPF (Stacking): After we save and reloading this DUT, it lose ospf vlan setting. |
| 11 | ECS4650EC-400 | OSPF (Stacking): After we change network type to broadcast from point-to-point; DUT print error message and ospf operation is failed. |
| 12 | ECS4650EC-380 | IGMP: Static groups cannot be showed immediately when add static groups. |

Runtime V1.1.0.9, Loader V0.0.1.0

| 1 | ECS4650EC-430 | Remove Stacking on ECS4650 |
| --- | --- | --- |

Runtime V1.1.0.10, Loader V0.0.1.0

| 1 | ECS4650EC-435 | sysLED blink issue. |
| --- | --- | --- |

Runtime V1.1.0.11, Loader V0.0.1.0

| 1 | ECS4650EC-440 | Can't save power inline maximum allocation settings |
| --- | --- | --- |
| 2 | ECS4650EC-439 | Disable power inline maximum allocation cause console halt |
| 3 | ECS4650EC-436 | There is something that is not working in MLAG. |

Runtime V1.1.0.12, Loader V0.0.1.0

| 1 | ECS4650EC-446 | [PoE] The maximum power allocation settings are different from CLI and WEB |
| --- | --- | --- |
| 2 | ECS4650EC-445 | [PoE] After reset maximum allocation to default, the maximum power allocation become 90W |
| 3 | ECS4650EC-442 | Enhancement logging command |

Runtime V2.4.0.252, Loader V0.0.1.0

| 1 | ECS4650EC-495 | ECS4650 support Policy Based Routing |
| --- | --- | --- |
| 2 | ECS4650EC-494 | ECS4650 support RIPng function |
| 3 | ECS4650EC-499 | Enhancement: support remote license |
| 4 | ECS4650EC-498 | Enhancement: support image profile license |
| 5 | ECS4650EC-497 | Enhancing the Radius feature can use Radius/TLS on Simba main to avoid CVE-2024-3596. |
| 6 | ECS4650EC-517 | ECS4650 upgrade openssl3.3 and openssh9.8 |

Runtime V2.4.1.252, Loader V0.0.1.0

| 1 | ECS4650EC-519 | [CraftPort-0030] Cannot setup IPv6 link-local address on craftport. |
| --- | --- | --- |
| 2 | ECS4650EC-526 | Syslog is flooded with SNMP following error messages : “Reject SNMP 127.0.0.1” |
| 3 | ECS4650EC-527 | [ECS4650-54T] No option to choose 25g media Type in Web |
| 4 | ECS4650EC-528 | GMP: The groups can be added when adding static groups exceeds the maximum number. |
| 5 | ECS4650EC-529 | SSH server disappear if to upgrade to phase2 version |
| 6 | ECS4650EC-530 | RIP: When receiving a 0.0.0.0 route with a subnet mask of 255.255.255.0, the DUT shall not update its routing table. However, the DUT currently does update the route table, which is incorrect behavior. |
| 7 | ECS4650EC-531 | PIM: DUT cannot select correct RPF neighbor sometimes. |
| 8 | ECS4650EC-532 | PBR: After this 1st_next-hop down; DUT cannot forward IP traffic to second next-hop. |
| 9 | ECS4650EC-533 | IGMP: The DUT still sends a G.S query after receiving the join report again. |
| 10 | ECS4650EC-534 | PBR: The route-map deny action would drop incoming packets. |
| 11 | ECS4650EC-535 | PBR: After we bind PBR with ACL dscp-set 0; DUT reload itself. |
| 12 | ECS4650EC-536 | PBR: After we save and rebooting this DUT; it lose mutiple sequence setting. |
| 13 | ECS4650EC-537 | OSPFv2: After we reload DUT and reset network type to broadcast from point-to-point. It prints error messages and ospf operation is failed. |
| 14 | ECS4650EC-538 | GMP per vlan 沒開的話, 打入multicast streaming , show igmp snooping group顯示異常 |
| 15 | ECS4650EC-539 | VRRP: The priority of VRRP is incorrect value: 281470681743460. |
| 16 | ECS4650EC-540 | RIP: The DUT cannot counter the RecBad Packets when send the invalid RIP version. |
| 17 | ECS4650EC-541 | Selective QinQ: The double-tagged configure of port will disappear when the port is changed to a trunk member. |
| 18 | ECS4650EC-542 | The DUT is unable to send metric=16 (Poisoned Reverse) update packets when using the default configuration of RIPng. |
| 19 | ECS4650EC-543 | igmp: FRR do not support igmp proxy ,but igmp proxy info already showed in cli show igmp interface . |

modifications:

Runtime V2.4.2.252, Loader V0.0.1.1

| 1 | ECS4650EC-450 | [PoE] The maximum power allocation settings are different from CLI and SNMP MIB |
| --- | --- | --- |
| 2 | ECS4650EC-468 | #37377 Enhance the extended HTTP/HTTPS/SSH timeout logout time for ECS4650 |
| 3 | ECS4650EC-472 | Add static mac in port security |
| 4 | ECS4650EC-482 | L3 Remove some inapplicable and incorrect command |
| 5 | ECS4650EC-518 | [Console&Telnet-0010] The DUT freezes after copy startup configuration file from TFTP server |
| 6 | ECS4650EC-525 | [ConfigFileMgmt] The port 51,52 will disappear when copy "Factory_Default_Config" to running-config. |
| 7 | ECS4650EC-544 | PoE: Class 5-8 can be set on ports 1-40 via WebUI. |
| 8 | ECS4650EC-546 | PBR: The DSCP value is changed when incoming packets match the ACL and the route-map is deny type. |
| 9 | ECS4650EC-548 | SFTP: The CLI of DUT will freezing and reboot after 10 mins when download the configure file by sftp session. |
| 10 | ECS4650EC-551 | RIPng: The maximum allowed value for the update timer is 65,535, despite the descriptive range showing up to 2,147,483,647. |
| 11 | ECS4650EC-553 | MLAG: The CLI of DUT will freeze and reboot after 10 minutes when setting a non-existent port-channel member in an MLAG group. |
| 12 | ECS4650EC-554 | OSPFv3: The instruction says to use 'clear ipv6 ospf6 process', but the executed command was 'clear ipv6 ospf process'. |
| 13 | ECS4650EC-556 | add replace C tag. |
| 14 | ECS4650EC-549 | RIPng: Cannot disable the ripng function |
| 15 | ECS4650EC-560 | When we create VLAN interface 20 and assign IP address to it,“interface vlan 20” appear twice in the running config |
| 16 | ECS4650EC-561 | The way to reset configuration or password without super password. |
| 17 | ECS4650EC-486 | Modify the uboot password. |
| 18 | ECS4650EC-572 | new feature : stop-only mode for accounting command |
| 19 | ECS4650EC-547 | PBR: The DSCP 0 disappeared when the route-map was bound to the interface. |

Runtime V2.4.3.253, Loader V0.0.1.1

| 1 | ECS4650EC-571 | Enhancemnet: Add gdb tools |
| --- | --- | --- |
| 2 | ECS4650EC-568 | close tls1.0 tls1.1 |
| 3 | ECS4650EC-562 | Need auto NAND recovery. |
| 4 | ECS4650EC-563 | MAC-vlan: DUT does not reject invalid MAC VLAN entry. |
| 5 | ECS4650EC-559 | Remove Stacking on ECS4650 |
| 6 | ECS4650EC-569 | After Build make_all -D, Linux shell still can use |
| 7 | ECS4650EC-570 | [Enhancement] Craft IP support to be set in Web |

Runtime V2.4.4.253, Loader V0.0.1.1

| 1 | ECS4650EC-575 | SelectiveQinQ: Need to add the double-tagged command for the ECS4650 project. |
| --- | --- | --- |
| 2 | ECS4650EC-573 | ECS4650 http upload faild |

Runtime V2.4.5.253, Loader V0.0.1.1

| 1 | ECS4650EC-577 | OSPFv2: Cannot delete ospf area in VLAN configuration. |
| --- | --- | --- |

Runtime V2.4.6.253, Loader V0.0.1.1

| 1 | ECS4650EC-581 | OSPFv3: Remove area 0.0.0.0 in VLAN; DUT delete all OSPF area setting. |
| --- | --- | --- |
| 2 | ECS4650EC-584 | OSPFv2: After we reload this DUTL it lose ospf vlan setting. |
| 3 | ECS4650EC-585 | OSPFv2: Please add virtual-link configuration setting. |
| 4 | ECS4650EC-590 | OSPFv3: DUT accept invalid router-id 0.0.0.0. |
| 5 | ECS4650EC-591 | add L3 DHCP VLAN has config failed |
| 6 | ECS4650EC-594 | after setting the ip ssh authentication-retries , the actual value will be reduced by one time |
| 7 | ECS4650EC-598 | mgmtd upgrade to V1.9.12-17625 |
| 8 | ECS4650EC-587 | OSPF v2: DUT cannot learn route from backbone area. |
| 9 | ECS4650EC-586 | OSPF v2: DUT cannot learn external route from its neighbor. |

Runtime V2.4.7.253, Loader V0.0.1.1

| 1 | ECS4650EC-599 | The CLI becomes unresponsive when executing the show running-config command. |
| --- | --- | --- |

Runtime V2.4.8.263, Loader V0.0.1.1

|  | ECS4650EC-633 | SSH continuously displays abnormal log messages |
| --- | --- | --- |
|  | ECS4650EC-632 | how ssh displays stale session entries after copy tftp running-config |
|  | ECS4650EC-631 | ConfigFileMgmt: Performing copy operations via the WebUI will keep the file system constantly busy. |
|  | ECS4650EC-630 | ssh connect and exec copy tftp running will cause crash |
|  | ECS4650EC-629 | Using MobaXterm_Personal ssh log in switch, segmetition fault occurs, and show log flash An exception occurs. |
|  | ECS4650EC-628 | ECS4650 Phase2 Upgrade openssh version to 10.4p1 |

Loader V0.0.0.6

|  | Initial Version |
| --- | --- |

Runtime V1.1.0.1

|  | Initial Version |
| --- | --- |

Work as Designed (wAD) eprs:

Known Bugs and Issues:

Any other problems than those listed above should be reported to our Technical Support Staff.

NA

Known RESTRICITIONS AND Limitations:

|  | Limitation Type | Description |
| --- | --- | --- |
| 1 | Software limitation | For verify chip can support full multicast entry. The speed of data/control packet which used for igmp snooping learning shall not in line speed. |
| 2 | Software limitation | Web Authentication only support IPv4 HTTP |
| 3 | Software limitation | Due to Jquery software limitation. The web browser need IE 9+, Chrome 54+, Firefox 52+ or Opera 41+. |
| 4 | Software limitation | Support muintuple vlan tag by software, but have limitation: can not support the packet need linux kernel process, like management features, arp, telnet, ping, tracerout, routing, snmp, ipv6…etc. [ES4627MB-FLF-EC-00889] |
| 5 | Chip limitation | ECS4150EC-48 DHCP-Snooping-0340：the rate limit of dhcp snooping does not work well According to the chip documentation, their speed limit is not 100% accurate, and the speed limit value is calculated by accumulating 10 seconds. |
| 6 | Chip limitation | Casued by Phy limit, can not support auto-negotiate of flow-control，We use force mode instead.So if link up with 100M/1G full，flow-control type should always Dot3X，10/100 half type should be backpress. |
| 7 | Phy chip limitation | ECS4150EC-63 The link flapping is caused by back-pressure and link would be up after back-pressure condition is gone(This Back-pessure only 100M/10M half on ECS4150-AC5) |
| 8 | Chip limitation | ECS4150EC-64 Per system, IC have 1 mirror buffer, after this packet is send out, it can be used for mirroring the next packet again. So you can not enable the mirroring, source, destination or multiple at ac5/ac3, ac3bxh solution. So when mirror two streams at same time,the mirror packets will loss |
| 9 | Chip limitation | After the SFP port is switched to 1G linkup after 10G linkup, if the traffic is blocked and the CLI command “show counter” displays “Internal Mac Transmit Errors / Fragments”, you need to unplug the port module or shutdown/no shutdown port to change the port link status, then the streaming will return to normal (generally, this problem will occur when the device is switched to 1G after the first 10G linkup after startup) |
| 10 | Chip limitation | Dynamic QoS Assignment only support configure one acl per port: Marvell support ACL resource are finite, so the upper software limit only support configure one acl per port or per function;when mac-authentication enable Dynamic Qos Assignment base Radius, so also only support configure one acl per port, multiple acls will authentication failure. |
| 11 | Chip limitation | AC5X does not support half-duplex |
| 12 | Software limitation | cpssDxChMirrorRxPortSet is called to set mirrored port when setting port mirror, and it is also called by sflow-sampling. So you can not set sflow-sampling and mirrored port on the same physical port at the same time. |
| 13 | Chip limitation | For AC5X devices, when a SFP interface works on 10G mode and there is streaming on it, and the down/up of the adjacent SFP interfaces will cause packet loss on the streaming interface. |
| 14 | Chip limitation | Cable-diag accuracy issues: For link-down ports, the reported distance to a fault is accurate to within +/- 2 meters. For link-up ports, the accuracy is +/- 10 meters. |
| 15 | Chip limitation | For AC5X devices, FDB does not support qos marking, and igmpsnp/mldsnp does not support priority function |
| 16 | Chip limitation | For AC5X, can not adapt the media-type of DAC cable(kr) and fiber(sr_lr) automatically for 49-54 ports. So add a CLI command for set it manually : Vty-1(config-if)#media-type sfp-forced ifmode ? kr Specifies forced ifmode as KR sr_lr Specifies forced ifmode as SR_LR The mode is fiber(sr_lr) by default, so if you need to insert DAC cable on a port, you have to set the mode to kr by this command. |
| 17 | Chip limitation | ECS4150EC-377: On Marverll chips, the output ratelimit is non-coninunous, only a specific set of values can be set. When we set a output ratelimit value, accturely we can only select a nearest value between these specific values. Because AC5 and AC5X belong to different chips, the set of values all different. For AC5X, when the value of output ratelimit is small (less than 637), the interval is 25. 75:64-87 100:88-112 125:113-137 150:138-162 175: 163-187 200: 188-212 225: 213-237 And so on 625: 613-637 When the ratelimit output value is larger than 637, the accuracy is within %2 For AC5, test according to the following specifications: When the value of output ratelimit is small (less than 2055), the interval is 30 or 31. 61: 64-90 91:91-120 121:121-151 152:152-181 182: 182-211 212: 212-241 242:242-271 272:272-302 And so on When the ratelimit output value is larger than 2055, the accuracy is within %2 |
| 18 | Chip limitation | ECS4650EC-45: Stacking: flow control across devices is not supported on AC5X chip |
| 19 | Software limitation | ECS4650EC-66: Stacking: For the ACL matching, we only support the case that the source port and the effective port on the same device. The source port matching will not take effect if it is on the different device with the effective port. |
| 20 | Chip limitation | ECS4150EC-409: Marvell currently does not have good support for half-duplex collision counting. |
| 21 | Chip limitation | CFM Can’t detect errorCCMdefect caused by mep ID in 1-3 interval level with hardware implementation, and In 1-3 interval level with hardware implementation can't support stacking. |
| 22 | Chip limitation | ECS4150EC-413: The rate limit is applied to all enabled types of storm traffic as a whole, meaning if certain rate limit is set and for example Unregistered Unicast and Broadcast are enabled for rate limit, it will be applied cumulatively to both types of traffic, not separately to each type. |

Compliance support:

| Compliance Level | Compliant |
| --- | --- |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2674 | Q-bridge | - dot1qBase group (1-5) - dot1qTp group (1-2) - dot1qStatic group(1) - dot1qVlan (1-5) (9-10) |
| V-Bridge MIB | V-Bridge MIB | - dot1vProtocol group |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |

| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |
| --- | --- | --- |
| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |

ECS4650 SERIES Private Enterprise MIB Support:

| Title | Version |
| --- | --- |
| ECS4650.mib | 202306190000Z |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

Customer Support:

By Phone:

By Email:

By Web:

By Fax: