Access Switch ECS4100-52T/ECS4100-26TX/ECS4100-28T/ECS4100-12T/ECS4100-28P/ECS4100-52P/ECS4100-12PH/ECS4100-28TC/ECS4100-12Tv2

Phase 3 Firmware Version 3.1.15.263

INTRODUCTION:

ECS4100-52T is a Gigabit Ethernet L3-Lite switch featuring 48 10/100/1000 ports, and 4 Gigabit Ethernet SFP uplink ports.

ECS4100-26TX is a Gigabit Ethernet L3-Lite switch featuring 24 10/100/1000 ports, and 2 10Gigabit Ethernet SFP+ uplink ports.

ECS4100-28T is a Gigabit Ethernet L3-Lite switch featuring 24 10/100/1000 ports, and 4 Gigabit Ethernet SFP uplink ports.

ECS4100-12T/12Tv2 is a Gigabit Ethernet L3-Lite switch featuring 8 10/100/1000 ports, 2 Gigabit Copper/SFP combo ports, and 2 Gigabit Ethernet SFP uplink ports.

ECS4100-12PH is a Gigabit Ethernet L3-Lite PoE switch featuring 8 10/100/1000 PoE ports, 2 Gigabit Copper/SFP combo ports, and 2 Gigabit Ethernet SFP uplink ports.

ECS4100-28TC/28TC-F is a Gigabit Ethernet L3-Lite switch featuring 24 10/100/1000 ports, and 4 Gigabit Copper/SFP combo ports.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| NTC Test Version | 3.1.15.263 | Git rtk_main (2023/11/13 10PM) | 2026/8/18 |
| NTC Test Version | 3.1.14.262 | Git rtk_main (2023/11/13 10PM) | 2026/6/16 |
| QA Test Version | 3.1.13.261 | Git rtk_main (2023/11/13 10PM) | 2026/1/21 |
| NTC/EIT/QA Test Version | 3.1.12.254 | Git rtk_main (2023/11/13 10PM) | 2025/12/20 |
| NTC Test Version | 3.1.11.252 | Git rtk_main (2023/11/13 10PM) | 2025/6/5 |
| EIT Test Version | 3.1.10.252 | Git rtk_main (2023/11/13 10PM) | 2025/5/22 |
| EIT&NTC Test Version | 3.1.9.252 | Git rtk_main (2023/11/13 10PM) | 2025/4/17 |
| EIT Test Version | 3.1.8.244 | Git rtk_main (2023/11/13 10PM) | 2025/02/14 |
| Cloud QA Test Version | 3.1.7.244 | Git DEV_ECS4100_Phase3 (2023/11/13 10PM) | 2024/11/27 |
| EIT Test Version | 3.1.6.244 | Git DEV_ECS4100_Phase3 (2023/11/13 10PM) | 2024/11/15 |
| EIT&NTC Test Version | 3.1.5.241 | Git DEV_ECS4100_Phase3 (2023/11/13 10PM) | 2024/3/28 |
| NTC Test Version | 3.1.4.241 | Git DEV_ECS4100_Phase3 (2023/11/13 10PM) | 2024/1/23 |
| NTC Test Version | 3.1.3.234 | Git DEV_ECS4100_Phase3 (2023/11/13 10PM) | 2023/12/4 |
| EIT Test Version | 3.1.2.234 | Git DEV_ECS4100_Phase3 (2023/11/13 10PM) | 2023/11/13 |
| NTC Test Version | 3.1.1.233 | Git DEV_ECS4100_Phase3 (2023/08/1 10PM) | 2023/9/18 |
| EIT Test Version | 1.3.3.233 | Git DEV_ECS4100_Phase3 (2023/08/1 10PM) | 2023/8/11 |
| NTC Test Version | 1.3.2.231 | Git Master (2023/03/7 10PM) | 2023/03/7 |
| EIT Test Version | 1.3.1.231 | Git Master (2023/02/17 10PM) | 2023/02/17 |

LOADER:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Testing Version | 0.0.4.0 | - | 2023/02/17 |
| Testing Version | 0.0.4.1 | - | 2023/03/7 |
| Testing Version | 0.0.4.2 | - | 2023/03/10 |
| Testing Version | 0.0.4.3 | - | 2023/08/11 |
| Testing Version | 0.0.4.4 | - | 2023/08/11 |
| Testing Version | 0.0.4.5 | - | 2023/12/04 |
| Testing Version | 0.0.4.6 | - | 2024/11/15 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Please see the ECS4100_ECS2110_Phase2_FeatureList_20180418.xlsx

Installation and Configuration Notes:

In general, the ECS4100 Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ECS4100 Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Known Bugs and Issues:

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

loader code modifications:

Loader V0.0.4.6

|  | EPR ECS4100EC-968 | eye diagram amplitude of PHY RTL8218D is too large on SDK4 ECS4100-12T/ECS4100-12Tv2/ECS4100-28T |
| --- | --- | --- |

Loader V0.0.4.5

|  | EPR ECS4100EC-537 | ECS4100_12T SDK4 uboot would auto reset for once when start up |
| --- | --- | --- |

Loader V0.0.4.4

|  | EPR ECS4100EC-537 | ECS4100_12T SDK4 uboot would auto reset for once when start up |
| --- | --- | --- |

Loader V0.0.4.3

|  | EPR ECS4100EC-537 | OAM:After cold reboot dut,cli print log of error on ECS4100-28TX |
| --- | --- | --- |

Loader V0.0.4.2

|  | EPR ECS4100EC-510 | ECS4100_12T port LED is incorrect |
| --- | --- | --- |

Loader V0.0.4.1

|  | EPR ECS4100EC-484 | Fix that 12T/28T print error information |
| --- | --- | --- |

Loader V0.0.4.0

|  | EPR N/A | Upgrade SDK to 4.0 |
| --- | --- | --- |

runtime code modifications:

(ECS4100) Runtime V 3.1.15.263, Loader V0.0.4.6

ECS4100_V3.1.15.263.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1263 | ssh connect and exec copy tftp running will cause crash | RD |
|  | ECS4100EC-1261 | Upgrade mgmtd to v1.9.23 on 4100 phase3 branch | RD |
|  | ECS4100EC-1260 | upgrade OpenSSH to 10.4p1 for ecs4100_26tx on BR_ECS4100_Phase3 | RD |

(ECS4100) Runtime V 3.1.14.262, Loader V0.0.4.6

ECS4100_V3.1.14.262.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1243 | Upgrade openssh version to 10.3p1 | RD |

(ECS4100) Runtime V 3.1.13.261, Loader V0.0.4.6

ECS4100_V3.1.13.261.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1217 | remove the remote license function to reduce the runtime size | RD |
|  | ECS4100EC-1211 | Reset Config: Failed to reset default configuration sometimes. | NTC |
|  | ECCLOUD-6686 | Mgmtd upgrade to 1.9.20-17701 | RD |

(ECS4100) Runtime V 3.1.12.254, Loader V0.0.4.6

ECS4100_V3.1.12.254.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1209 | Mgmtd upgrade to V 1.9.18-17678 | RD |
|  | ECS4100EC-1209 | remove the default snmp community （public and private） | RD |
|  | ECS4100EC-1170 | LGU+ loop when 4100 soft reset | TS |
|  | ECS4100EC-1202 | Device rebooted when configuring the running-config by customer settings | TS |
|  | ECS4100EC-1091 | Support remote license function on rtk_main branch | RD |
|  | ECS4100EC-1135 | Enhance: Please add the snmp trap for MAC_thrashing alarm. | RD |
|  | ECS4100EC-1134 | Configuration: Change startup Opcode and reload this DUT; it return error message. | NTC |

(ECS4100) Runtime V 3.1.11.252, Loader V0.0.4.6

ECS4100_V3.1.11.252.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECCLOUD-5399 | Device constantly has an out of space issues, cannot create new files. | RD |
|  | ECS4100EC-1132 | Enhancement： MAC_thrashing alarm request to include mac and VLAN | RD |

(ECS4100) Runtime V 3.1.10.252, Loader V0.0.4.6

ECS4100_V3.1.10.252.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1096 | Enhancing the Radius feature can use Radius/TLS on Simba main to avoid CVE-2024-3596. | RD |
|  | ECCLOUD-5468 | After downgrade device cannot register to the Cloud. | RD |
|  | ECS4100EC-1079 | Static LACP not removed correctly and admin-key anomaly. | TS |
|  | ECS4100EC-989 | ENhancement：LLDP remote information will display with MAC not interfaces information.(rtk_main) | RD |
|  | ECS4100EC-1105 | modify the compare version No. | RD |
|  | ECS4100EC-1104 | Enhance: Alarm to generate the Smart-pair | RD |
|  | ECS4100EC-1113 | Enhance: alarm to generate when the Tx/Rx power and Temperature crosses beyond the threshold [rtk_main] | RD |
|  | ECS4100EC-1115 | Enhance DHCP option 82 setting remote ID from global mode to port interface.[rtk_main] | RD |
|  | ECS4100EC-1133 | It's stuck for 5 minutes while doing show running config with specific configure | TS |

(ECS4100) Runtime V 3.1.9.252, Loader V0.0.4.6

ECS4100_V3.1.9.252.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1069 | MIB: The mib fie is miss option by "fileAutoUpgradeMgt" | RD |
|  | ECS4100EC-1085 | upgrade openssl to 1.1.1w | RD |
|  | ECS4100EC-1088 | Without backdoor - Remove runtime backdoor 、Linux backdoor and super account. | RD |
|  | ECS4100EC-1063 | Irregular Operation: DUT can reload when firmware transmission(issue of sequence) | RD |
|  | ECCLOUD-5258, ECCLOUD-5251 | Save flash space, fix version upgrade | RD |
|  | ECS4100EC-1095 | disable tls1.0 tls1.1 for openssl 1.1.1w | RD |
|  | ECS4100EC-946 | SelectiveQinQ: Configuration on the port-channel is missingafter reboot. | NTC |
|  | ECS4100EC-1062 | Irregular Operation: DUT can reload when firmware transmission(issue of sequence) | NTC |
|  | ECS4100EC-1108 | NTP: The DUT will freezing and reboot when added and delete the ntp server more times. | TS |

(ECS4100) Runtime V 3.1.8.244, Loader V0.0.4.6

ECS4100_V3.1.8.244.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1052 | openssl/openssh upgrade. | RD |
|  | ECS4100EC-1065 | It shows "General Error" when try to set mib oid trunkLinkDelayDown" | RD |

(ECS4100) Runtime V 3.1.7.244, Loader V0.0.4.6

ECS4100_V3.1.7.244.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-1024 | ECS4100 Phase3 can not set dying gasp from cloud | RD |

(ECS4100) Runtime V 3.1.6.244, Loader V0.0.4.6

ECS4100_V3.1.6.244.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V3.1.6.241.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-968 | eye diagram amplitude of PHY RTL8218D is too large on SDK4 ECS4100-12T/ECS4100-12Tv2/ECS4100-28T | RD |
|  | ECS4100EC-973 | formal license can not work | RD |
|  | ECS4125EC-659 | Increase password complexity | TS |
|  | ECS4125EC-666 | log need record the user password change event | TS |
|  | ECS4100EC-1004 | Upgrade mgmtd to v1.8.33-17466 for 4100&2100&2110&4125 PBE | RD |
|  | ECS4100EC-1006 | TCP port 5151 will be scanned as open port via NMAP. | TS |

(ECS4100) Runtime V 3.1.5.241, Loader V0.0.4.5

ECS4100_V3.1.5.241.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V3.1.5.241.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-822 | Additional SNMP communities adoptd_private and doptd_public | TS |
|  | ECS4100EC-814 | adjust amplitude value of PHY RTL8218D to make eye diagram better | RD |
|  | ECS4100EC-812 | [IPv4-RemotePing-0260]DUT doesn't reply ICMP TimeStamp Request. | EIT |
|  | ECS4100EC-889 | DDM: "!" does not show after plugging in both transceiver and fiber even transceiver-monitor is enabled | NTC |
|  | ECS4100EC-757 | CableDiag: Pair C and D display "NS" is better than "NT" when it's connected to 10/100Mbps. | NTC |
|  | ECS4100EC-842 | Add profile license and bypass feature for 4100-26tx/12ph | RD |
|  | ECS4100EC-840 | 1. Support image-profile function on ECS4100-12PH & ECS4100-26TX. 2. Support bypass function to copy runtime without suppport image-profile directly. | RD |
|  | ECS4100EC-922 | Enhancement: Add the no port-utilization period command line | TS |
|  | ECS4100EC-921 | MIB DUT not support link-delay | RD |

(ECS4100) Runtime V 3.1.4.241, Loader V0.0.4.5

ECS4100_V3.1.4.241.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V3.1.4.241.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-899 | ECS4100-Phase3 can not recognition the old license | TS |

(ECS4100) Runtime V 3.1.3.234, Loader V0.0.4.5

ECS4100_V3.1.3.234.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V3.1.3.234.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-802 | DDM:"!" is still existed even transceiver-monitor is disabled on SDK4 | RD |
|  | ECS4100EC-740 | cant't forward packets and watchdog reboot. | RD |
|  | ECS4100EC-745 | MVR: DUT down when set mvr domain 1 type source | NTC |
|  | ECS2100EC-270 | snmp-server notify-filter not working | RD |
|  | ECS4100EC-820 | ECS4100 Mac address table missing for ecloud(ECCLOUD-185) | RD |
|  | ECS4100EC-822 | Additional SNMP communities adoptd_private and adoptd_public for ECS4100 phase3 | TS |
|  | ECS4100EC-643 | Enhancement:Modify port description length from 64 to 128 | Enhancement |
|  | ECS4100EC-779 | add PHY RTL8218D support for ECS4100 SDK4 runtime | RD |

(ECS4100) Runtime V 3.1.2.234, Loader V0.0.4.4

ECS4100_V3.1.2.234.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V3.1.2.234.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-716 | Rate limit configuration can't be removed from running config. | TS |
|  | ECS4100EC-709 | establish a script to build image which contains uboot and runtime | RD |
|  | ECS4100EC-728 | L2 DHCP Relay: DUT does not reject to set invalid DHCP realy address with network_ID. | NTC |
|  | ECS4100EC-751 | Enhancement: The add command is used to view the status of the spanning tree added to a specified VLAN. | Enhancement |
|  | ECS4100EC-752 | BPDU statistics is not visible and not able to find per vlan mstp root port | Enhancement |
|  | ECS4100EC-749 | Ping: CLI only supports 64 characters length of target hostname. | Enhancement |
|  | ECS4100EC-746 | Enhancement: Terminal Monitor functionality to monitor the console logs | Enhancement |
|  | ECS4100EC-747 | Enhancement: cannot alter the interface load interval | Enhancement |
|  | ECS4100EC-742 | Control-plane protection(ICMP/SSH/Telnet in interface) for anomolys | Enhancement |
|  | ECS4100EC-741 | Need support Access-list on cpu port so customer can deny packet to cpu. | Enhancement |
|  | ECS4100EC-755 | "Copy sftp file" does not list log. | NTC |
|  | ECS4100EC-748 | Enhancement: add the below commands for IPv6 source-guard sdp and sip/sdp on ECS4100. | Enhancement |
|  | ECS4100EC-766 | Enhancement:Customer request to have cvid vlan_range configuration support on ECS4100 | Enhancement |
|  | ECS4100EC-782 | enhance the ping payload size enlarge to 1472 which the IP_MTU will be 1500 in ip packet(for SDK4) | Enhancement |
|  | ECS4100EC-781 | (SDK4)hold-off timer functionality to delay the interface transition from up to down and vise versa &22.we need port dampening functionality to avoid the interface flaps | Enhancement |
|  | ECS4100EC-780 | Option to release the port from shut to no shut once storm is stopped | Enhancement |
|  | ECS4100EC-775 | config grep(sh run ¦ i) to filter the config specific to the strings or numbers etc | Enhancement |
|  | ECS4100EC-771 | Enhancement:Tacacs authorization failover requirement. | Enhancement |
|  | ECS4100EC-764 | add PHY RTL8218D support for ECS4100 SDK4 uboot | RD |
|  | ECS4100EC-794 | ImageMgmt: The same version shall be set to "Failed". | NTC |
|  | ECS4100EC-773 | Not able to default the port configurations | Enhancement |
|  | ECS4100EC-772 | We need the rapid ping count for maximum 10000 | Enhancement |
|  | ECS4100EC-774 | Not able to configure the interface parameters with interface range. | Enhancement |

(ECS4100) Runtime V 3.1.1.233, Loader V0.0.4.4

ECS4100_V3.1.1.233.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V3.1.1.233.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-701 | [RateLimit-0150, 0161, 0162] unknow config limit DUT RX frame rate to 500. | EIT |
|  | ECS4100EC-702 | Statistics-0090, 0100] multicast counter is not correct | EIT |
|  | ECS4100EC-711 | DHCPv6 Relay: Configure dhcpv6 client and Change DHCPv6 relay destination address to multicast-all form vlan 1 and system halted | RD |
|  | ECS4100EC-712 | After creating the VLAN4094 show running will display an error message | NTC |
|  | ECS4100EC-704 | [VLAN-MAC-Based-0060] SYS_ADPT_MAX_NBR_OF_MAC_VLAN_ENTRY support 256 but test result is 128 | EIT |
|  | ECS4100EC-692 | [CPU-Guard-0010] MIN_cpuGuardHighWatermark should be 40, not 20 | EIT |
|  | ECS4100EC-691 | modify the https certificates,using sha256 signature for 4100,2100,2110 | NTC |
|  | ECS4100EC-695 | [IPv6-AddrConfig-0612,614]no support show ipv6 default-gateway | EIT |
|  | ECS4100EC-694 | [IPv6-AddrConfig-0280] DAD operation fail | EIT |
|  | ECS4100EC-681 | ECS4100_12T SDK4 uboot would auto reset for once when start up | RD |
|  | ECS4100EC-673 | [LACP-140, 150] Multicast have rate-limit with 500 pps in default configuration. | EIT |
|  | ECS4100EC-672 | ECS4100 phase3 can not use the old license | RD |
|  | ECS4100EC-680 | File system is abnormal after update runtime from SDK2 to SDK4 | RD |
|  | ECS4100EC-697 | [MAC-Filter-0080] MAC filter was limited rate after reboot | EIT |
|  | ECS4100EC-689 | [1QVLAN-0350, 1QVLAN-0390, 1QVLAN-1491] disable IGMP snooping but multicast was not forwarded | EIT |

(ECS4100) Runtime V 1.3.3.233, Loader V0.0.4.3

ECS4100_V1.3.3.231.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V1.3.3.231.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-496 | RMON: Got big counters after clear counters | NTC |
|  | ECS4100EC-481 | reduce runtime image size since it's increased when rx los interrupt is enabled | RD |
|  | ECS4100EC-510 | ECS4100_12T Port LED is incorrect | RD |
|  | ECS4100EC-502 | DHCP-Server: client-identifier is useless. | NTC |
|  | ECS4100EC-490 | ATC-0080: ATC does not work | EIT |
|  | ECS4100EC-501 | WEB: Lose this L2 Relay and L3 Relay help page. | NTC |
|  | ECS4100EC-509 | WEB: Failed to enter into Relay Help page. | NTC |
|  | ECS4100EC-516 | Web: Incorrect keyword in DHCPv6-SNP Help page. | NTC |
|  | ECS4100EC-480 | ECS4100_52P takes long time to startup | RD |
|  | ECS4100EC-522 | The maximum DNS cache entry is not 6400 | NTC |
|  | ECS4100EC-521 | ImageMgmt: Run "http_upload" from WebUI or "copy ftp/tftp/sftp file", and then "reload" from CLI/Web immediately, it shall be rejected. | NTC |
|  | ECS4100EC-507 | WEB: This maximum static route number is incorrect in Help page. | NTC |
|  | ECS4100EC-489 | IGMP-Filter-0020 : information lost in igmp filter table | EIT |
|  | ECS4100EC-531 | SNTP IPv6: Some invalid IPv6 address can be accepted | NTC |
|  | ECS4100EC-508 | DHCP Relay: DUT accept invalid DHCP relay setting with network_ID and itself IP. | NTC |
|  | ECS4100EC-530 | DDM: DDM information is incorrect and unreasonable | NTC |
|  | ECS4100EC-541 | LLDP: Save this lldp setting of Trunk; leave port from trunk, it cannot transmit and receiving lldp. | TS |
|  | ECS4100EC-540 | LLDP: After we reset this lldp notify interval from maximum field value, DUT cannot send out notify trap message. | TS |
|  | ECS4100EC-525 | Cable diagnostics shall not be displayed "IE(0)" on pair A and B when it's connected to 10/100Mbps. | NTC |
|  | ECS4100EC-552 | Commandlogfile cannot be generated | TS |
|  | ECS4100EC-526 | UDLD: MIB file has no udld trap notification. | NTC |
|  | ECS4100EC-499 | CLI shows unknown messages after reload the DUT. | NTC |
|  | ECS4100EC-528 | SystemMgmt: CPU temperature raise/reduce 16 degrees one time | NTC |
|  | ECS4100EC-559 | [SDK4]ECS4100_12TV2 keeps restarting | NTC |
|  | ECS4100EC-482 | high CPU usage on ECS4100_12Tv2 | RD |
|  | ECS4100EC-634 | [CLI] ECS4100_28T/ECS4100_28TC should not support OAM dying gasp | RD |
|  | ECS4100EC-635 | system up time over 496 days will reset[sdk4.0] | TS |
|  | ECS4100EC-537 | OAM: After cold reboot DUT, CLI print lot of error message and port can't linkup on ECS4100_28TC. | NTC |
|  | ECS4100EC-495 | MGMTD:there wiil be a logt of logs likes "Reject SNMP 127.0.0.1" when enable mgmtd | TS |
|  | ECS4100EC-661 | RMON History: Counter can't cleared(show rmon history) after IXIA stop sending and wait 30s to update status | NTC |
|  | ECS4100EC-555 | ipaddrprimaryinterface and iPaddrstatus code can not get correct data | RD |
|  | ECS4100EC-665 | Static Route: After we add and delete some static routes, it cannot create maximum 32 routes on DUT. | NTC |
|  | ECS4100EC-664 | IP default-gateway is changed after rebooting the switch.[SDK4] | TS |
|  | ECS4100EC-511 | DHCP-Server: Client ID can not be set correctly in WebUI. | NTC |

(ECS4100) Runtime V1.3.2.231, Loader V0.0.4.1

ECS4100_V1.3.2.231.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V1.3.2.231.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-484 | ECS4100 12T/28T print error information | RD |
|  | ECS4100EC-492 | There are error informations like "Error, can not read from PoE Error" no none POE device | RD |
|  | ECS4100EC-488 | Diffserv-0910: queue mode of strict does not work | EIT |
|  | ECS4100EC-487 | QinQ-0040 0060 0070:tagged Packet Forwarding on Dot1Q Tunnel Port not work well | EIT |
|  | ECS4100EC-486 | COS-830,850 : queue mode Strict dosen't work | EIT |

(ECS4100) Runtime V1.3.1.231, Loader V0.0.4.0

ECS4100_V1.3.1.231.bix is for 52T/26TX/28T/12T/28P/52P/28TC/12Tv2

ECS4100-12PH_V1.3.1.231.bix is for 12PH

|  | EPR | Headline | Role |
| --- | --- | --- | --- |
|  | ECS4100EC-335 | ECS4100 update sdk4.0 | RD |
|  | ECS4100EC-337 | ECS4100-12PH update sdk4.0 | RD |
|  | ECS4100EC-344 | Ping doesn't work on SDK4 | RD |
|  | ECS4100EC-341 | ECS4100 sdk4 LED cannot be lit | RD |
|  | ECS4100EC-367 | ECS4100-12tv2 can not noraml startup | RD |
|  | ECS4100EC-363 | failed to bind port - channel to ethernet | RD |
|  | ECS4100EC-371 | ACL startup and configuration error. | RD |
|  | ECS4100EC-381 | SDK4 DIAG:No multicast diag command. | RD |
|  | ECS4100EC-380 | Rate limit and meter: The rate limit mode is changed from L1 to L2. | RD |
|  | ECS4100EC-396 | If a port joined multiple VLAN, it only first VLAN_ID will be listed of the log when the port is down. | RD |
|  | ECS4100EC-398 | Disable tsl 1.0 because a security breach was detected. | RD |
|  | ECS4100EC-408 | WatchDog: "ip tftp timeout 900" will trigger watchdog reboot when TFTP server is not existed. | RD |
|  | ECS4100EC-399 | 802.1ag-CFM-1410: DUT will reboot when we input cfm command in telnet | RD |
|  | ECS4100EC-400 | IPV6 Source Guard: The static IPv6 SG entry cannot replace this DHCP-SNP and ND-SNP entry. | RD |
|  | ECS4100EC-401 | IPv6_Route: Default gateway can not be changed when DUT enables autoconfig and receives RA. | RD |
|  | ECS4100EC-409 | CLI: Irregular operation via SFTP is different if cable is disconnected when transferred | RD |
|  | ECS4100EC-410 | The timeout value (1-540) of tacacs is too long that will be caused the watchdog event.Radius: The timeout value (1-65535) of radius is too long that will be caused the watchdog event | RD |
|  | ECS4100EC-412 | MIB: DUT does not support this DHCPv6 Relay MIB. | RD |
|  | ECS4100EC-413 | MIB: Cannot get this MIB objects "dhcpv6RelayServerInetAddrEntry" and "dhcpv6RelayServerVlanEntry". | RD |
|  | ECS4100EC-414 | DHCPv6 Snooping: DUT does not support interface-id and remote-id policy configuration by WEB. | RD |
|  | ECS4100EC-402 | MSTP-1150 : DUT no response when Set DUT1 and DUT2 mst name | RD |
|  | ECS4100EC-403 | DHCPv6 Relay: After we delete and add DHCPv6 Relay, DUT cannot send out DHCPv6 RelayForw to Server. | RD |
|  | ECS4100EC-404 | DDM: High Warning & High Alarm shall be triggered when temperature is higher than the value. | RD |
|  | ECS4100EC-405 | 192.168.2.x could not be used when default VLAN 1 is DHCP. | RD |
|  | ECS4100EC-411 | ssh\telnet client support backdoor | RD |
|  | ECS4100EC-425 | Voice VLAN:DUT does not reject invalid OUI setting with itself MAC | RD |
|  | ECS4100EC-418 | Voice VLAN: This voice phone still join to voice vlan as disable lldp rule | RD |
|  | ECS4100EC-419 | Voice VLAN: DUT dooes not reject this invalid OUI MAC | RD |
|  | ECS4100EC-416 | Cluster member snmp trap and snmp connect send failed | RD |
|  | ECS4100EC-423 | HTTP-HTTPS:The idie time will reset automatic, the session can no logout | RD |
|  | ECS4100EC-422 | http-https:the web page management of dut will cut off when using the chrome to access the https session | RD |
|  | ECS4100EC-427 | MVR6:source port can not join groups after join groups and clear groups | RD |
|  | ECS4100EC-428 | MVR6:Receiver port can be changed to access port. | RD |
|  | ECS4100EC-430 | MVR6:source ipv6 address of group-specific query is wrong when set IPv6 address to mvr6 vlan | RD |
|  | ECS4100EC-429 | MVR6:IPv6-AddrConfig:Link-local address shows "INVALID". | RD |
|  | ECS4100EC-434 | MVR6:Debug:status of mvr6 debug command can not be showed in cli. | RD |
|  | ECS4100EC-415 | MIB: DUT does not support this dhcpv6DynamicProvisionStatus MIB | RD |
|  | ECS4100EC-436 | WEB: DUT does not support DHCPv6 dynamic provision configuration page. | RD |
|  | ECS4100EC-438 | MVR6:receiver port can not forward multicast data which DIPd are not in MVR6 profile when mldsnp is enabled. | RD |
|  | ECS4100EC-439 | Web: Loss DHCPv6 description in Help page. | RD |
|  | ECS4100EC-442 | Cluster: The SNMP trap relayed by commander always be coldstart trap | RD |
|  | ECS4100EC-444 | ACL counter error: The acl does not display or clear the hit counts | RD |
|  | ECS4100EC-441 | VLAN-translation: Ingress-filtering could not be startup after reboot | RD |
|  | ECS4100EC-445 | DHCPv6 Relay: DUT does not reject to set dhcpv6 relay setting as ipv6 address of vlan1 not defined. | RD |
|  | ECS4100EC-446 | DHCPv6 Relay: After we change DHCPv6 relay destination address to multicast-all; DUT cannot send out RelayForw packets. | RD |
|  | ECS4100EC-447 | DHCPv6 Relay: DUT does not reject to set reserved ipv6 address as relay destination address. | RD |
|  | ECS4100EC-449 | MVR6:Query received in receiver ports would flood to othter receiver ports. | RD |
|  | ECS4100EC-443 | MVR6:MLDSNP is effected when MVR6 is enabled. | RD |
|  | ECS4100EC-455 | VLAN.The port outbound vlan behavior is incorrect | RD |
|  | ECS4100EC-336 | ECS4100 reduces the size of kernel and runtime. | RD |
|  | ECS4100EC-456 | Rate limit: dos-protection echo-chargen and Policy-map rate limit invalid. | RD |
|  | ECS4100EC-458 | MVR6:the sourc prot without mrouter port sends a registered group report when DUT received general query. | RD |
|  | ECS4100EC-467 | MAC-Thrashing:MAC-Thrashing-0440: Can't save MAC-Thrashing configuration. | RD |
|  | ECS4100EC-468 | Display error msg: "In function 'dal_cypress_l2_addr_add' Error code:0xF029" when Reload the DUT after MAC-Thrashing configuration. | RD |
|  | ECS4100EC-377 | Jumbo frame: Enable jumbo frame failed for ECS4100 upgrade sdk4. | RD |
|  | ECS4100EC-472 | L2PT: Set "dot1q-tunnel system-tunnel-control" failed for ECS4100 upgrade sdk4. | RD |
|  | ECS4100EC-459 | MVR6:source port forwards received query to all other source ports and receiver ports when mldsnp mrouter port is set. | RD |
|  | ECS4100EC-479 | driver_proc crashed after update runtime | RD |

Known RESTRICITIONS AND Limitations:

| 1 | L2 table (16K entries) share for L2 unicast, L2 multicast, IPv4 multicast, IPv6 multicast, Routing. |
| --- | --- |
| 2 | For all Basic Switching Unit-Test items of the half duplex, the counter of fragments would increase the same number/speed as the counter, collisions on IXIA. It is confirmed by Realtek Josh. It is chip normal behaviour which follows the standard. RTK use the “Collision based backpressure” of flow control (backpressure) on half mode. It will send the jam pattern packet to do the flow control. |
| 3 | Due to chip limitation, do not support Port-based QoS. |
| 4 | Chip limitation: Not supported for global ACK, which was known as “VLAN ACL” before. |
| 5 | Chip limitation: MAC ACL does not allow for configuring any IP/IPv6 pattern. |
| 6 | Chip limitation: Supports for mapping CoS/DSCP to PHB (or Internal Priority) and then to Queue. But CoS/DSCP-to-PHB mapping also changes a packet's trap-to-cpu queue. Therefore, SW design decides to remove CoS/DSCP-to-PHB mapping, causing straight CoS/DSCP-to-Queue mapping on user configuration. [ECS2000-28PP-00344] Note: The implementation gets to be partially compliant with RFC 2597/2598. Implemented the CLI command: “Console(config-if)#[no] qos map {cos-queue ¦ dscp-queue}”, for ethernet inferface only. |
| 7 | Chip limitation: Ingress/Egress DiffServ poorly supports for SRTCM/TRTCM remarking. 1) Meter burst size is global setting, instead of per-police. => So hard to meet various SRTCM/TRTCM cases. 2) Ingress rule does not support for color action and CoS/DSCP remarking. => Has to additionally use egress rule. 3) Egress ACL action is global for DLF/broadcast/multicast traffic, however, per-port for unicast. => DLF/broadcast/multicast will not be matched with port list. Therefore, SW design decides to only implements flow rate-limiting. [ECS2110-26T-00034] Implemented the CLI command: “Console(config-pmap-c)#[no] police rate <16-10000000 Kbps>”. Besides, DiffServ ingress policy-map can remark CoS only. If a police is configured to remark DSCP, it shall be rejected while binding to input port. [ECS2110-26T-00034] [Workaround in ECS4100-52T-00123] |
| 8 | Chip limitation: Following counters are not supported. These counters are not visible. [ECS2000-28PP-00051,00466] IF-MIB: Discard Input (ifInDiscards) Unknown Protocols Input (ifInUnknownProtos) Error Output (ifOutErrors) - RTL838x does not support this counter, however, RTL839x does. QLen Output (ifOutQLen) EtherLike-MIB: Alignment Errors (dot3StatsAlignmentErrors) SQE Test Errors (dot3StatsSQETestErrors) Internal Mac Receive Errors (dot3StatsInternalMacReceiveErrors) Carrier Sense Errors (dot3StatsCarrierSenseErrors) |
| 9 | Chip limitation: Cable diagnostics can detect cable length up to 100 meters with an inaccuracy around 15 meters. |
| 10 | Chip limitation: Jumbo frame is supported by system, instead of per port. |
| 11 | Chip limitation: Voice VLAN can not support for remarking DSCP priority (because rule action can not remark CoS and DSCP at the same time). [ECS2000-28PP-00041] |
| 12 | Chip and HW board limitations: When an SFP transceiver is inserted, CPU will take some more time to have it initialized. This will lead to slow system response. |
| 13 | Chip limitation: Not supported for false carrier events ‘rpMauFalseCarriers’ in MAU-MIB. The counter value will always stay in 0. |
| 14 | Chip limitation: The collision mac-address is not affordable. |
| 15 | Chip limitation: Not supported for changing the priority of multicast traffic in IGMPSNP/MVR/MVR6. [ECS2000-28PP-00341, ECS2110-26T-00064,76,129] |
| 16 | Chip limitation: One single reason for trap-to-cpu of protocol packets, like BPDU/LACP/LLDP/dot1X (because all of these packets go through queue #5). |
| 17 | Chip limitation: IPv4 standard ACL and IPv4 extended ACL are implemented using a same rule slice. Their entries share the total 128 rules in the slice. |
| 18 | Chip limitation: IPv6 standard ACL and IPv6 extended ACL are implemented using a same rule slice. Their entries share the total 64 rules in the slice. |
| 19 | Chip limitation: Not supported for GVRP bypass VLAN check in RTL839x, but supported in RTL838x. (Supported for BPDU/LACP/LLDP bypass VLAN check in 8x [ECS2000-28PP-000147]. 9x as well.). Therefore, when a receive port does not join PVID VLAN and its VLAN ingress filter is enabled, GVRP does not work (GVRP packet is dropped by chip). |
| 20 | Chip limitation: MAC-based VLAN can not work with IP Source Guard. [ECS2000-28PP-00321] |
| 21 | Chip limitation: Internal loopback test does not work on 10G SFP+ ports (port# 25,26) in ECS2110-26T device because of no PHY chip there. |
| 22 | Chip limitation: When a port is linkup at 10 Mbps speed, cable diagnostics is not supported. [ECS2110-26T-00055] Btw, cable-diag can only measure cable length on the two conditions: 1. (Link_Up; Speed_1000M) and 2. (Link_Down; link partner inexistent, port admin up). Case_1) Link_Up 1.1) Speed_1000M => can measure cable length 1.2) Speed_100M => cannot measure cable length 1.3) Speed_10M => cannot measure cable length (cable-diag is not supported actually) Case_2) Link_Down 2.1) link partner existent, port admin up => cannot measure cable length 2.2) link partner existent, port admin down => cannot measure cable length 2.3) link partner inexistent, port admin up => can measure cable length 2.4) link partner inexistent, port admin down => cannot measure cable length |
| 23 | Chip limitation: L2 hashing algorithm (algo_1) will get to observable collisions when both Source MAC address and VLAN ID are incremental successively. However, when only SA is random (no matter if VID is fixed, incremental, or random), L2 hashing will have very good performance. |
| 24 | Chip limitation: L2 table can contain up to (16K + 64) entries because BCAM (extension table with additional 64 entries) is enabled. [ECS2110-26T-00211, ECS2100-52T-00032] |
| 25 | ECS4100-28T Board limitation: 1) Not available for SFP_Present state (GPIO signal). 1-1) SW is not aware of any SFP insertion, and does not read out the EEPROM data. So, not able to afford any SFP information (CLI “show interfaces transceiver”). 1-2) When 100M SFP is inserted and connected to link partner, the fiber port will not get to linkup until it is forced to 100FX mode (CLI “media-type sfp-forced 100fx”). * No such issues (1-1, 1-2) in SW V1.1.5.164 and higher. 2) System and Port LEDs are single-color and can only be lighting in green (blinking or solid). 3) System LED will not be blinking while system is booting. 4) There is no thermal sensor for system temperature. Fan neither. |
| 26 | Chip limitation: When a packet is mirrored to mirroring port due to one port’s rx and the other port’s tx, only one packet (i.e. the copy of that rx) will be copied to mirroring port. [ECS4100-52T-00046] |
| 27 | Chip limitation: There is only 1 copy, which is from the port with smallest port id, for mirroring non-unicast in egress direction(tx). [ECS2100-10P-ACN-00022] |
| 28 | Chip limitation: When an input packet's field (e.g. VLAN_ID) is modified by ingress ACL, egress ACL is not able to match that updated value because it still sees the original packet's value. [ECS4100-52T-00138] For example, 1. port_1 is configured with ingress VLAN translation 10 -> 100, port_2 is configured with egress VLAN translation 100 -> 200. 2. Inject tagged packets with VLAN_ID=10 to port_1. 3. These packets will not out-go from port_2 because port_2 sees VLAN_ID=10, instead of VLAN_ID=100. |
| 29 | Chip limitation: For the tagged traffic between QinQ uplink port and normal port, if QinQ TPID is not 0x8100, packet's egress TPID will be incorrect. [ECS4100-52T-00195] PS. #00138 workaround solution is disabled, however, #00094 workaround solution is enabled. For example, QinQ TPID is 9100. When 9100 tagged packet is incoming from QinQ uplink port and outgoing to normal port with tagged member, its TPID remains 9100. (Should be 8100.) And vice versa. Say, 8100 tagged packet, normal port income, QinQ uplink port outgo, TPID remains 100. |
| 30 | ECS4100-28T/28P/28TC Board limitation: There is no GPIO signal available for SFP_Present state. Therefore, in order for SFP-insertion polling, it takes more CPU resource to read SFP EEPROM data via I2C channel. This may result in higher CPU utilization, as compared with other skus having GPIO signal for SFP_Present. |
| 31 | Chip limitation: The ARP packet would be still filtered by MAC ACL even the ACE permits the source MAC address. The cause is Realtek chip matches ‘Source MAC’ /ACE with ‘Sender Hardware Address’ /ARP. The two fields of an ARP packet are supposed to be the same. However, they are different for some reason. [ECS4100-52T-00247] |
| 32 | Chip limitation: ATC (Auto Traffic Control) does not support for Unknown Unicast storm control. [ECS4100-52T-00258] |
| 33 | Chip limitation: ACL and DiffServ can be bound to a same egress port. However, it gets higher priority to match ACL. [ECS4100-52T-00427] |
| 34 | Chip limitation: RTL839x does not have the ifInDiscards /IF-MIB register. However, SW works around to provide a substitute ifInDiscards counter, and its valus is a sum of (dot1dTpPortInDiscards, rxMacDiscards) registers. [ECS4100-52T-00629] - dot1dTpPortInDiscards: count of valid frames received which were discarded (i.e., filtered) by the forwarding process (dropped in ALE) - rxMacDiscards: the number of packets dropped in MAC RX and switch core including bad packets and dropped packet due to lack of resource (dropped before ALE) |
| 35 | SW limitation: When both VLAN Trunking and Ingress Filtering are enabled, the system will drop those to-CPU packets, carrying unknown VLANs (not existent in the system). For example, Enabled both VLAN Trunking and Ingress Filtering, a DAI trusted port does not forward unknown-VLAN ARP packets. Note: First has a port's Lookback Detection disabled, then can have its Ingress Filtering disabled. |
| 36 | Notes for ECS4100-12PH supporting 60W UPoE (802.3bt Type 3 Class 5~6), starting from V1.2.37.191 [ECS4100-52T-00634] 1. Broadcom PoE MCU firmware version has to be upgraded to 4.0.0.5 first. 2. Any PD shall follow 3bt/at/af specifications of current and voltage ratings. If the ratings are exceeded due to short circuits or voltage transients, PSE ports might be damaged permanently, as per Broadcom PAE advices. 3. So far Accton PoE Loader (AnalogDevice LT4321) is tested, and the PDs can be provided power up to 60W. That means no other chip vendor’s 3bt PD solution is guaranteed for 60W. 4. When 3 PDs are provided with power 58.x~60.x W each (causing the total power consumption get 175+ W), 12PH PoE powering will become unstable - some PD may lose the power in several(3) minutes. |
| 37 | Chip limitation: When the software reinstalls the Dying Gasp packet, the old dying Gasp packet will not be released and will continue to occupy the packet buffer, when a long time it will use all of the packet buffer, and cause the chip can not send and recive any packet.So when the mac,output port of the Dying Fasp packet has no change, software will not reinstalls the packet;and the Dying Gasp packet can be reinstalls 1024 times.When use dying gasp with trunk,the dying gasp will send from all merber ports of the trunk. |

Compliance support:

| Compliance Level | Compliant |
| --- | --- |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2674 | Q-bridge | - dot1qBase group (1-5) - dot1qTp group (1-2) - dot1qStatic group(1) - dot1qVlan (1-5) (9-10) |
| V-Bridge MIB | V-Bridge MIB | - dot1vProtocol group |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |

| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |
| --- | --- | --- |
| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |

ECS4100 Private Enterprise MIB Support:

| Title | Version |
| --- | --- |
| ECS4100-Series.mib | 201903080000Z |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

CUSTOMER SUPPORT:

By Phone:

By Email:

By Web:

By Fax:

Port LED Definitions:

The following Port LED definitions are effective in Runtime V1.2.19.182 and higher.

| Mode \ Port LED \ Sku | Link/Act |  |  |  | PoE | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1000 Mbps | 10/100 Mbps | Loopback-Detection |  |  |  |
|  |  |  | Copper | Fiber / Combo |  |  |
| ECS4100-28T (12) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | NA | > Reset button - Yes(12) > LED - Bi-color |
| ECS4100-28P (13,6) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | Orange | > Reset button - Yes(13), No(6) > LED - Bi-color |
| ECS4100-12T (14,9) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | NA | > Reset button - Yes(14), No(9) > LED - Bi-color |
| ECS4100-12PH (10) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | Orange | > Reset button - No > LED - Bi-color |
| ECS4100-52T (4) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | NA | > Reset button - No > LED - Bi-color |
| ECS4100-52P (8) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | Orange | > Reset button - No > LED - Bi-color |
| ECS4100-26TX (1,2) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | NA | > Reset button - No > LED - Bi-color |
| ECS4100-28TC (15,11) | Green | Orange | Green<0.5>Orange | Green<0.5>Orange | NA | > Reset button - Yes(15), No(11) > LED - Bi-color |
| ECS4100-28T (3) | Green | Green | Green | Green | NA | > Reset button - No > LED - Single-color |

PS1. The value(s) within parentheses are the associating Board ID(s).

PS2. The Port LED light of a looped port on loopback-detection only applies to the port-disabled mode (disabled port’s Tx/Rx traffic).

PS3. Green<0.5>Orange means for alternately Green/Orange flashing in 0.5sec period, say |<- Green,0.5s ->|<- Orange,0.5s ->|<- Green,0.5s ->|<- Orange,0.5s ->|...|<- Green,0.5s ->|<- Orange,0.5s ->|.

PS4. At PoE mode Port LED light will not be blinking - either solid (powering) or off (not powering).

PS5. For ECS4100-26TX (1,2), when 10G SFP Port 25/26 get to linkup at 10G speed, the Port LED is also lighting in Green (same as 1000M).

Device Name v.s. 1st Firmware Version:

Below given is the ECS4100 Device Name and the 1st Runtime Version, which is operational in that device.

| Device | BID | 1st Runtime Ver | 1st Loader Ver | Note |
| --- | --- | --- | --- | --- |
| 12Tv2 | 17 | 1.2.27.183 | 1.0.2.2 | 12T board relayout, ext.GPIO -> CPLD |
| 28TC-F | 16 | 1.2.26.182 | 1.0.2.0 |  |
| 28TC-ITUS | 15 | 1.2.19.182 | 1.0.1.8 |  |
| 12T-ITUS | 14 | 1.2.16.181 | 1.0.1.7 |  |
| 28P-ITUS | 13 | 1.2.16.181 | 1.0.1.6 |  |
| 28T-ITUS | 12 | 1.2.16.181 | 1.0.1.6 | Replaced with 28TC-ITUS |
| 28TC | 11 | 1.2.16.181 | 1.0.1.6 |  |
| 12PH | 10 | 1.2.13.174 | 1.0.1.1 |  |
| 12T | 9 | 1.2.2.172 | 1.0.1.1 |  |
| 52P | 8 | 1.2.2.172 | 1.0.1.1 |  |
| 52T | 7 | NA |  | Deposed |
| 28P | 6 | 1.2.2.172 | 1.0.1.1 |  |
| 28T | 5 | NA |  | Deposed |
| 52T | 4 | 1.1.8.164 | 1.0.0.5 |  |
| 28T | 3 | 1.1.8.164 | 1.0.0.5 |  |
| 26TX-ME | 2 | 1.1.8.164 | 1.0.0.5 |  |
| 26TX | 1 | 1.1.8.164 | 1.0.0.5 |  |
| 52T | 0 | 1.1.8.164 | 1.0.0.5 |  |